Time-dependent reliability assessment framework against nonstationary extreme hazard induced by climate change, all in a Bayesian setting

Overall instruction:

Current study is aiming to establish the novelty, research relevance and significance in “time-dependent reliability assessment methodological framework against nonstationary extreme coastal hazard driven by process-informed covariates and climate change in an Bayesian setting.” The study aims to understand the research gap in solving the unknowns of deriving nonstationary extreme hazard which is primarily done by using only time as covariate, whereas this study aims to climate process drives and propagate that into structural reliability. Further detail are provided in subtopic “0. Intention”. However even doing so the limitation of this study are that it considers time-dependent reliability problem but without consideration of deterioration focusing just on hazard evolution based on climate state and failure probability dependent on those hazard state. Also the study limits the shape parameter to be constant since it is too sensitive to validate with only 80 years of data. Also the covariates right now are the climate indices defining the climatic state and climate variability but the climate change are not introduced as sea level rise covariate but rather change in meteorological condition in itself. Also this is life-cycle reliability problem and no risk metrics is considered in this research. For every section you should be very careful about the instruction: (1) “1. Introduction”: you should conduct systematic literature review to establish the relevance, need, significance and novelty of using “process-informed covariates and BAMLSS for nonstationary hazard, why using this approach would solve current research challenges, provide novel and significant understanding which is crucial for the time-dependent reliability assessment of structures”. (2) “2. Methodology”: make sure the framework is specific to the study but portrayed as an general framework with applicability and not as a case study. (3) “4. Illustrative example”: Make this section as case study where the specific condition like study area, structure adopted in this study as well as the result and discussion. (4) “5. Conclusions”:Make sure the conclusion reflects the need we establish in methodology and justifies the new understanding in four novel conclusion points. (5) “Abstract and title”: After you complete all the section, based on the content write this section.

Additional point to remember: please don’t describe any limitation as shortcomings of the study but rather try to convince based on relevant literature why despite being necessary, in this study it is out of scope or irrelevant which can be further investigated if required. Make sure every consideration and limitation is justifiable and convinced (not just hypothetically but backed with existing academic literature on the topic). Since this is an academic writing for publishing scientific paper, be very careful, THE STATEMENT SHOULD BE CLEAR, CONCISE, ALIGN WITH THE STUDY AND NOT VAGUE OR GENERAL, ADEQUATE USE OF TECHNICAL TERMS BUT NOT TOO OVERWHELMING WITH THE JARGONS. Meaning: if readers read the statement they understand this study and not get information which are irrelevant to understand the topic, except for some of the information section where general understanding might be required between few places.

Citation and reference style should be followed as for journal like structural safety and Elsevier, similar to the style of this paper.

Also all the content in this study where just writing style or information flow needs to be taken as a reference are highlighted in “blue”, you just follow the writing style and how the statement structure, flow of information and content should be writing. But the portion highlighted in “yellow” can be used for writing the current study which can be improvsed if required but remember those content are aligned to the study.

- Intention:

Answer: which climate covariates causes failure probability growth? Is the increase due to location change? Is it due to scale change? Which climate state produces highest failure probability? How much reliability is lost if climate covariates are ignored?

You start from:

Ht∼GEV(μt,σt,ξ)

but instead of directly calculating annual failure probability, you explicitly integrate the fragility function:

Pf(t)=∫P(F∣h)fH(h,t)dh

Then:

PLC=1-t=1T(1-Pf(t))

You care about how the evolving hazard distribution interacts with fragility. Reliability is the result of that interaction.

If your framework can quantify:

Covariate→(μ,σ)→HazardChanges→ReliabilityChanges

and decompose the reliability loss into identifiable climate-process drivers,

then it becomes substantially more interesting. This paper does not merely estimate future failure probability; it explains the mechanism by which climate-process nonstationarity propagates into structural reliability

Minimum metrics you should derive

For your paper, I would recommend these as the core set:

- μ(t), σ(t)from BAMLSS

- Time-varying exceedance probability P(Ht>hc)

- Time-varying return level zT(t)

- Annual failure probability Pf(t)

- Life-cycle failure probability PLC

- Expected number of exceedances/failures

- Design-life level

- Covariate contribution to Pf(t)

10. Covariate sensitivity / contribution metrics

This is very important for novelty.

You should quantify:

∂μ∂Xi,∂σ∂Xi

and then:

∂Pf∂Xi

This tells:

Which climate covariate most increases failure probability?

For example:

- ENSO mainly changes μ

- IPO mainly changes σ

- sea-level rise shifts μ

- combined ENSO + SLR produces highest Pf(t)

This is where your framework becomes more than just a curve.

So derive a decomposition like:

| Source | Effect on hazard | Effect on failure probability |
| --- | --- | --- |
| ENSO | changes μ | increases/decreases annual Pf |
| IPO | changes σ | changes tail spread |
| SLR | shifts μupward | raises baseline Pf |
| ENSO + SLR | combined high state | produces reliability-critical years |

This will support the claim that your research gives process-informed reliability understanding, not only nonstationary hazard estimation.

Abstract

Instruction: after finalizing all the other section, prepare the standard abstract relevant to the study. But make sure the writing style and flow of information is structured as follows:

Riverine floods have become increasingly prevalent on a global scale, posing significant risks to infrastructure systems and communities. The escalating impacts of climate change associated with the increase in rainfall intensities and frequencies necessitate the improvement of the existing design methodologies to account for the non-stationary climate change effects to ensure that the target reliability is achieved and mitigate future flood disasters. This paper presents a novel load and resistance factor design approach for river embankments subjected to extreme rainfall under non-stationary climate change effects. The novel load and resistance factor design is proposed by introducing an additional partial factor to account for the effects of climate change. Future precipitation and temperature projections are collected from various climate models considering several cases of emission scenarios. A Quantile Delta Mapping is employed as a bias correction method to create more refined and locally accurate climate information from a global-scale climate model. An integrated hydrological and hydraulic modeling of the analyzed river are carried out to estimate the associated time-variant river discharge and water surface elevation. Based on the time-variant water surface elevation, a non-stationary extreme value theory associated with the maximum flood level is leveraged using the peak-over-threshold approach. The embankment reliability and the corresponding most probable points are evaluated using limit states associated with overtopping and slope failures. Based on the estimated and target reliability index, the design point for each random variable is assessed for both with and without climate change effects. Finally, the partial factors to consider climate change effects are determined as the ratio between the design points with and without climate change effects. As an illustrative example, the proposed framework is applied to the Ashida River in Fukuyama city of Japan, which is considerably vulnerable to riverine flooding. The effects of climate change on the reliability of the embankment and the associated partial factors are discussed.

Keywords: process-informed covariates, climate change, BAMLSS, time-dependent reliability, coastal hazard

# Overall general framework: only in illustrative example it should be case specific

# 1. Introduction

Instruction for writing the introduction: Make sure the introduction revolves around establishing the need, relevance, significance and novelty of our research and also methodology. With the systematic literature review of Q1 international scientific journal papers of the relevant field try to address why this research is a challenge until now, why understanding through our research objective and research method is necessary and how exactly without it currently there is gap in understanding the time-dependent reliability and why it is crucial (all statement should be backed by the existing literature, academic journal papers). The limitation should not be portrayed here and only the current study significance and relevance and relevance should be focused on.

The flow of information like starting from real damage from coastal hazard in the past can be presented, climate change impact then can be projected and then time-dependent reliability current understanding, current challenges, unknowns and establishing the need of this research and finally explaining the research flow can be described. The last paragraph to describe this research can be derived as a summary of all the method used from the methodology section. the use of language as grammar or writing structure and flow of information can be understood and taken as reference from blue highlighted section and then with the scientific literature for my study can be written. Make sure atleast 40-45 relevant citations from relevant journal papers and research articles be provided and make sure the content are relevant and not imaginatory. You can prioritize but not limit to journals like Reliability engineering and system safety, Structural safety, Natural hazards, Structures and infrastructure engineering, Ocean engineering, Coastal engineering, International Journal for Disaster Risk Reduction, ASCE and similar.

Natural hazards pose significant threats to public safety, infrastructure integrity, natural resources, and economic development around the globe. In recent years, floods have increasingly become a global concern, posing a significant threat to infrastructure and communities worldwide [1–3]. Notably, Japan has been one of the regions most susceptible to flood-related risks, with a substantial portion of these events attributed to embankment failures. For instance, in September 2000, the heavy rainfall event in the Tokai district of Aichi Prefecture caused an overtopping of the embankment and inflicted 63,440 buildings, with 5.5 million people evacuated [4]. Similar catastrophic flood disasters stemming from embankment failures were also recorded in Niigata Prefecture in July 2004 [5], Kanto region in 2015 [6], and northern Kyushu in July 2017 [7]. More recently, in 2018, the western part of Japan was afflicted by an exceptionally high rainfall amount that caused embankment failures along the river near the residential area. This event caused damage to over 2,000 buildings, resulted in 212 casualties, and triggered 619 geological disasters across 31 prefectures, marking it as one of Japan’s most significant flood events since 1982 [8,9]. In recent years, the frequency and impacts of extremes have increased and have been reported in substantially in many parts of the word such as Netherlands [10], Indonesia [11], and Bangladesh [12]. These historical events indicate that riverine floods have been becoming more severe and coastal transportation infrastructures are increasingly expected to deliver reliability service under environmental extremes whose statistical properties in the future due to climate change are evolving. For bridges in coastal settings, this evolution is not only a capacity or safety issue but also a serviceability problem due to enhance and alter intensity and frequency of coastal extremes against operational disruptions over the design life, challenging the conventional assumption that design environmental actions and associated reliability checks are time-invariant. For this reason, there is a great deal of interest in understanding how extreme events will change in the future. Therefore, it is essential to design embankments with adequate reliability that can withstand the increasing intensity and frequency of future riverine flood events.

In a nonstationary model, however the parameters of the underlying probability distribution function change over time or in response to a given covariate. At the process level, coastal flooding losses are primarily driven by extreme sea levels, for which climate change is expected to increase extremes, yet the evolution of key components (notably storm surge contributions) remain uncertain. To mitigate riverine flood disasters, rainfall assessment is paramount since it forms the basis for evaluating hydraulic loads and overall stability associated with embankments.Reliability based approaches have therefore been advanced for climate adaptation of infrastructure, including time-dependent reliability assessment and LRFD-type adjustments intended to keep reliability above target levels under nonstationary flooding. For coastal flood defences in particular, time-dependent reliability is necessary because climate-driven increases in hydraulic demand act simultaneously with deterioration-driven loss of resistance, producing a systematic decline in reliability over service time if left unamanged. However, Matalas (1997) argued that trend in hydrological records cannot firmly be established because of the variables intrinsic variability and limited length of observations. In his reasoning, the observed trend might only be part of a slow oscillation. Consequently, Matalas (1997) defined hydrological trends as “real (physical)” or “perceived (statistical)”. Even though using statistical trend analysis tools inevitably leads to detecting only statistical trends, it is important to make a distinction between a trend which has a physical explanation (e.g., increases in runoff in response to urbanization) and a trend which cannot be fully explained by our understanding of the underlying processes. Regardless of the type of observed hydrologic trends, i.e., in response to a physical process or only perceived (statistical), these trends challenge the stationary assumption (Milly et al., 2008).

One way to physically interpret is that variability of precipitation was found to be influenced by tht natural variability in the coupled oceanic-atmospheric system. To characterize ocean-atmosphere interactions, influences of global climatic indices on precipitation extremes and floods are attracting increasing attentions. To quantify the effect of climate indices on the occurrence frequency of extreme precipitation, many studies tended to incorporate climate indices, such as ENSO and North Altantic oscillation (NAO), as external climate forcing into models for frequency analysis. Despite these advances, a central obstacle remains: nonstationary hazard modeling is only as credible as the identification and representation of the physical drivers that govern changing extremes. A systematic review of nonstationary extreme value analysis in coastal settings highlights persistent challenges arising from data limitations, covariate choice, and dependence modeling i.e., the very components that determine whether nonstationarity is physically interpretable or merely statistical trend fitting. Broader reviews of nonsatioanry hydroclimatic extremes similary emphasize uncertainties due to insufficient record length, spurious nonstationarities, and incomplete representation of nonstaionary cources in modeling frameworks. Process-informed frameworks such as ProNEVA directly respond by integrating physical drivers into extreme value analysis and providing Bayesian uncertainty assessment, but they also highlight a fundamental difficulty: observed trends can be partly indistinguishable from intrinsic variability (“perceived” versus physically explained trends), so the attribution and selection of drivers cannot be treated as a resolved step in the workflow. Extensive efforts have been made to develop a the probabilistic rainfall model to assess the safety of embankments and geological hazards [13–19]. In response to the influence of climate change, various studies have introduced non-stationary extreme rainfall analyses [20–23]. Based on the probabilistic rainfall assessment, the hydrological and hydraulic integrated modeling approach was commonly adopted for embankment safety assessment using widely available tools such as the Soil and Water Assessment Tool (SWAT) and the Hydrologic Engineering Center River Analysis System (HEC-RAS) [24–27]. In addition, several studies have projected future riverine flooding scenarios and their potential intensification due to climate change using the aforementioned integrated approach [28–31]. However, these studies have not yet been implemented to develop appropriate embankment design methodologies considering climate change.

Coastal-infrastructure systems: dikes and seawalls, ports, bridges, transit networks, and associated lifelines are increasingly governed by climate amplified coastal hazards. The latest climate risk synthesis underscores that low lying coastal cities and settlements are already experiencing adverse impacts superimposed on accelerating anthropogenic coastal change and that compound and cascading risks to infrastructure and supply chains are expected to increase. However, this hazard-side advances have not been systematically converted into adaptive serviceability target reliablilty functions (or modulation rules) suited to designers operating under SSP-dependent futures. This creates a second, design-critical challenge: even with process-informed nonstationary modeling, engineers rarely know which drivers will dominate when, how many interacting drivers must be represented, and how large their combined effect on the tail behavior wil be over the design working life. Moreover, adopting a nonstationary model often requires assumptions about future process evolution, which adds to overall uncertainty; in other words, nonstationarity can reduce bias in principle while increasing epistemic uncertainty in practice when future evolution cannot bt credible specified. In coastal flooding, this tension is sharpened by low confidence in future trends of  storm surge contributions to extreme sea levels, placing a hard limit on how sharply future hazard trajectories can be “pinned down” from first principles alone. Consequently, setting reliability targetes as single fixed values becomes problematic: if the hazard trajectory is ambiguous, a point target may appear precise while being conditional on unstestable assumptions. In its early stages of development, embankment and other geotechnical design methods were primarily based on allowable stress design (ASD) [32–35]. Although ASD afforded a certain level of safety, ASD may occasionally lead to a design without sufficient reliability due to the exclusion of the limit states associated with serviceability and collapse. The transition from ASD to probabilistic assessment of embankment became an uprising topic in the early 1970s after introducing the first-order reliability method (FORM) [36–40]. The probabilistic approach of embankment design through load and resistance factor design (LRFD) leads to more efficient use of materials. Recently, the LRFD methodology has been developed and adopted for embankment by regulatory agencies such as the Missouri Department of Transportation [41,42].

This hazard evolution challenges the foundations of conventional design practice. Existing code-based design targets typically framed through stationary load maps, fized return periods, and implicitly calibrated reliability  indices are not guaranteed to maintain their intended safety amrgins when hazard frequency and intensity evolve over time. In time variant load process formulations developed for a changing climate, classical quantities such as return period and exceedance probability become time-dependent, motivating the conclusion that stationary design concepts are no longer adequate and that codes and standards require revision to preserve consistent levels of safety. Despite many efforts made to quantify the time-variant reliability of structures, particularly for embankment and slope structures [43–45], the non-stationary effects of climate change associated with increasing intensity and frequency of extreme hazards have not yet been considered in the design methodology of structures. Probabilistic embankment design can be carried out based on Monte Carlo simulation (MCS) [46,47]. Nonetheless, the MCS approach involves intricate computations, consumes an excessive amount of time, and is significantly impractical. Furthermore, the diverse temperature and rainfall projections from climate models and emission scenarios contribute to significant uncertainties in assessing future climate change impacts, in which many of these climate models exhibit discrepancies between the simulated climate conditions and the observed local climate characteristics [48,49]. Considering these challenges, a more practical design method is necessary through the development of LRFD methodology on structures under climate change effects to enhance future disaster preparedness.

Studies have also demonstrated that climate-informed calibration can be operationalized in code-making via load scaling factors calibrated to target reliability measures over the design working life, illustrating that reliability-consistent adaptation to nonstationary hazards is feasible in principle. However, translating nonstationary coastal hazard science into actionable target reliability guidance for bridge serviceability remains underdeveloped. Servicaeability target reliability is itself an active topic: reent work shows that codified annual target reliability indices for serviceability vary materially depending on whether they are derived via generic cost-optimization or back-calibration, and further notes that the rationale for back-calibration is often not documented, leaving uncertainty regarding which targets are cost-optimal or representative of practice

This paper addresses this gap by developing an integrated climate-to-reliability calibration framework for bridge serviceability in coastal environments. Forst, nonstationary storm surge and wave hazards are modeled using Bayesian distributional regression within the BAMLSS framework, which supports flexible additive covariate effects n distribution parameters and is weel sutied to multi-covariate, time-varying extremes modeling. Seconf, the covariate-driven, scenario-conditioned hazard uncertainties are propagated into time-dependent serviceability reliability to quantify reliability drift unders SSP2-4.5 and SSP5-8.5 forcing, explicitly isolating the load-side nonstationarity effect. Thirs, results are synthesized into an adaptive target-reliabilty modulation rule calibrated by reference to an initial target so that pracitioners can select climate-consistent serviceability target (or target ranges) as  afunction of time horizon and scenario, thereby converting nonstationary coastal hazard dynamics into design-relevant reliability guidance rather that post hoc reliability diasgnostics. This study addresses these coupled gaps by combinig distributional regression and adaptive reliability-based design. Specifically, BAMLSS enable probabilistic modeling of the entire conditional hazard distribution (rather than only its mean), while providing a Bayesian framework for uncretaitny quantification that is directly compatible with reliability assessment and the propagation of uncertainty into reliability bounds. Building on the adaptatiaon-apthways logic where signposts and triggers guide decisions as the future unfolds that study further proposes an adaptive reliability target rule that links observable or scenario-consistent hazard evolution patterns to reliability levels, thereby supporting context dependent acceptable risk and enabling engineers to select targets that are robust to both nonstationary change and deep uncertainty.

This paper presents a novel framework for the LRFD methodology of river embankments by introducing an additional partial factor to accommodate the non-stationary impacts of climate change on embankment performance. Future precipitation and temperature projections are collected from various climate models considering several emission scenarios. To enhance the accuracy of climate data derived from global-scale climate models, Quantile Delta Mapping (QDM) is employed as a bias correction technique, allowing the refinement of the information for local conditions. An integrated hydrological and hydraulic analysis of the target river is then conducted to estimate the river discharge and water surface elevation over time. Based on the time-varying water surface elevation data, a non-stationary extreme value theory associated with the maximum water surface exceeding the river cross-section height (hereafter referred to as the maximum flood depth) is applied through the peak-over-threshold (POT) approach. The  reliabilities of the embankment associated with the overtopping and slope failure modes are assessed. To illustrate the effectiveness of the proposed framework, an embankment in the Ashida River in Fukuyama city, Japan, is used as a study case. The impact of climate change on the embankment reliability and the corresponding partial factors are discussed.

# 2. Methodology

# 2.1 Overall Procedure for estimating time-dependent reliability assessment considering non-stationary climate change effects

Instruction for writing the 2.1 subsection: This subsection provides the overall methodological framework procedure as summary. Please make sure the statement of information provided or written here are not repeated again in the other subsection when we go in detail as per the sub section of methodology. You can add few points in the final as the limitation of the study but make sure not to describe any limitation as shortcomings of the study but rather try to convince based why despite being necessary elsewhere, in this study it is out of scope or irrelevant which can be further investigated if required, even if required here current version is sufficient and if readers want they can expand. You can refer to the current “blue” highlighted section to understand how it can be written like flow of information and then once every subsection is written, based on that information you can summarize both contextually and if needed mathmetically here. Also I have attached an image of flowchart for your reference, please provide me similar overall flowchart for this study in a similar context and mathematical equation, you can suggest image where necessary within the blocks of flowchart or it can be completely contextual and with relevant symbols and equation as well.

What this study basically does is derived process informed covariates, then numerically derived storm surge, then using BAMLSS determine the nonstationary extreme hazard analysis, conduct empirical fragility analysis based on statistical/empirical fragility curve, then determine the time-dependent failure probability  (as described in 0. Intention as above)

Various studies have suggested that the changing climate would lead to more frequent and intense riverine flooding [50–52]. Considering the increasing flood exposure associated with future populations in low-lying river regions [53,54], the potential consequences of these floods could be substantial. However, current research primarily focuses on assessing the impact of climate change on the embankment vulnerability [55–57]. In a recent study, Beber et al. [51] proposed a climate change adaptation strategy for river embankments by focusing on materials with low hydraulic conductivity. One limitation of this approach is assuming a stationary maximum water level. Due to the effects of climate change, streamflow and the associated maximum water level are expected to increase with time. As a result, the increase in maximum water level would make the embankment more susceptible to failure due to overtopping and slope instability. Therefore, considering non-stationary climate change effects in embankment design methods, such as introducing additional partial factors in LRFD approaches, is necessary to prevent future flood disasters.

Fig. 1 shows the procedure to determine the partial factors for embankment design considering non-stationary climate change effects by introducing a LRFD framework. The method comprises five main steps: 1) data collection of future rainfall intensity and temperature from several climate models, 2) hydrologic and watershed modeling of the analyzed river, 3a) hydraulic modeling of river flow in the analyzed river, 3b) probabilistic modelling of water level excess, 4) reliability assessment of embankment, and 5) estimation of the partial factors for embankment design based on LRFD concept considering climate change. The details are described as follows.

In step (1), climate models, specifically General Circulation Models (GCMs) from the Coupled Model Intercomparison Project Phase 6 (CMIP6) [58], are collected under various climate change scenarios known as Shared Socioeconomic Pathways (SSPs) [59]. GCMs simulate large-scale global climate patterns but have limitations due to coarse spatial resolution (typically 100 to 300 kilometers between grid points), hindering the capture of local-scale variations [60]. To address this, bias correction and statistical downscaling methods are applied. These methods establish relationships between local climate station observations and historical GCM data, enabling the derivation of bias-corrected data for future climate projections. Detailed discussions of bias correction and statistical downscaling techniques are presented in Section 3.

Based on the bias-corrected temperature and precipitation data, the hydrological assessment of the river section is carried out in step (2). SWAT and ArcGIS are employed, utilizing geospatial data such as a digital elevation model (DEM), land use information, and soil type data to define the watershed boundary and assess Hydrological Response Units (HRUs) [61]. Step (3a) involves a one-dimensional hydraulic analysis using HEC-RAS simulation software from the United States Army Corps of Engineers [62]. The river geometry in the embankment is defined based on DEM data, and the unsteady flow analysis is performed using the river discharge output from step (2). In this study, the flood depth Hexc is defined as the difference between the water surface elevation above the river cross-section depth hr and the ground level (see Fig. 2) [63]. Therefore, the maximum flood depth Hexc,max should be evaluated to ensure that the embankment can safely contain floodwaters and prevent overtopping events. Given the projected increase in flood intensity and frequency due to climate change, step (3b) introduces a method to combine POT using the non-stationary generalized Pareto distribution and extreme value analysis to estimate the time-variant distribution of the maximum flood depth from the present time t0 to an analyzed time tD (i.e., FHexc,max(hexc,[t0, tD])). The details of steps (2) and (3) are described in Section 3.

In step (4), the reliability assessment associated with the embankment design parameters is evaluated considering two failure modes: overtopping and slope instability. The reliability of the embankment is significantly influenced by its geometry, particularly the phreatic line associated with the river’s water level and the critical failure surface of the embankment. As a result, the limit state function of slope instability becomes increasingly complex. Therefore, in step (5), the partial factors associated with the embankment design variables are evaluated using the expanding ellipsoid approach introduced by Low and Tang [64], which is considered more beneficial in addressing the increasingly complex limit state function. Based on the evaluated reliability index, the design point for each embankment parameter is estimated in the proposed framework. The partial factors to ensure the safety of embankment against overtopping and slope instability can be defined as:

where Rk and Sk are the characteristic values associated with structural resistance R and load actions S, and γRa and γSa are the reduction and amplification factors related to R and S, respectively, which can be expressed as:

where RD* and SD* are the design point for R and S, respectively. In this study, the Rk and Sk characteristics value is assumed to be similar to their mean value [65].

Furthermore, additional partial factors are necessary to transform the design point RD* and SD* into new design points RD,cc* and SD,cc* to account for climate change effects. The LRFD expression in Eq. (1) becomes:

where:

Intuitively, the partial factors γRb and γSb measure the extent to which the design point should be modified with respect to the original design point RD* and SD* due to climate change, as illustrated in step (5). The detailed procedure to determine the partial factors for embankment considering non-stationary climate change effects is discussed in Section 4.

## 2.2. Statistical downscaling and bias correction of climate models

Instruction for writing the 2.2 subsection: The method described in this subsection as highlighted in “yellow” are used in this study. So you should just make sure the statement are upto the required standard of academic writing, also the information aligns with the overall methodological framework as well as connect with the next. You can refer to citation “3.1.0 and 3.1.2” if you need further methodological clarity before deciding. You can add other references where “citation” is mentioned based on the existing academic papers. Please check the mathematical equation, symbols, as a researcher does it make sense and is it mathematically correct and does it describe the context mathmetically correctly? Also make sure the symbol used here to represent any variable or condition if later repeated in other section then it should be consistent but if make sure same symbol for describing two different variable or condition should have their unique but clear representation and not overlapping the same one. If needed refer to similar existing scientific paper from the internet as well as content of this paper to understand how equation should be written in standard form and specific to this study, as well as if the context should be expanded or reduced.  Make this a proper methodological section, establish the relevance and need of this step briefly, then explain what are we doing contextually and mathematically, how this step contributes to the overall methodological framework and finally how this step will be connect to next one or next relevant one. Add more citation only if you feel its necessary. The citation mentioned in number and their corresponding file are uploaded in github, which you can refer for clarity of this method/step.

Make sure the equation numbers are correct as well both in numbering equation as well as intext mention.

Global climate models (GCMs) often contain systematic biases due to their coarse spatial resolution which results in imperfect representation of regional climate characteristics [Citation]. To address such biases, the Quantile Mapping (QM) method is commonly employed by percentile mapping the similar statistical characteristics between observed historical data  and modeled historical data[ Citation]. To extend its application for future climatic signals , Quantile Delta Mapping (QDM) is utilized as complementary to QM, for preserving the model-projected relative changes and correcting future distributional biases. Using a percentile-based method, the empirical cumulative density function (CDF) associated with the observed historical data from the local climate station and the simulated historical and future projection data from GCMs, are estimated for a specific month k. Generally, the GCM data according to the QDM method can be described as Eq. 7 [ 3.1.0]:

where  denote the bias-corrected meteorological variable (e.g., sea level pressure and wind speed) associated with future projections from GCMs at time t; t = {tyear, tmonth, tday} is defined as a date vector where tmonth are the time variables expressed for each month k; is the relative changes between the historical and future data of climate variables in the GCM output;  serves as the placeholder binary operator that is later defined depending on the relative change formulation as explained in Eqs. 8 & 9.

Depending upon the physical characteristics of the meteorological variables, the systematic bias correction as explain in Eq. 7 becomes either additive or multiplicative [3.1.2]. For those variables, whose relative changes are physically interpreted as absolute difference, (e.g., sea-level pressure), additive QDM is used:

In addition, for climatic variables with a physically meaningful zero lower bound (e.g., wind speed), the distortion of low magnitude values are avoided through multiplicative QDM:

The resulting bias-corrected climate variables were then utilized as meteorlogical forcing input for numerical modeling of surges using ADvanced CIRCulation (ADCIRC) model.

## 2.3. Numerical modeling of coastal surge, flooding using ADCRIC

Instruction for writing the 2.3 subsection: The method described in this subsection as highlighted in “yellow” are used in this study. So you should just make sure the statement are upto the required standard of academic writing, also the information aligns with the overall methodological framework as well as connect with the next. You can refer to citation “3.1.3” if you need further methodological clarity before deciding. You can add other references where “citation” is mentioned based on the existing academic papers. Please check the mathematical equation, symbols, as a researcher does it make sense and is it mathematically correct and does it describe the context mathmetically correctly? Also make sure the symbol used here to represent any variable or condition if later repeated in other section then it should be consistent but if make sure same symbol for describing two different variable or condition should have their unique but clear representation and not overlapping the same one. If needed refer to similar existing scientific paper from the internet as well as content of this paper to understand how equation should be written in standard form and specific to this study, as well as if the context should be expanded or reduced. Make this a proper methodological section, establish the relevance and need of this step briefly, then explain what are we doing contextually and mathematically, how this step contributes to the overall methodological framework and finally how this step will be connect to next one or next relevant one. Add more citation only if you feel its necessary. The citation mentioned in number and their corresponding file are uploaded in github, which you can refer for clarity of this method/step.  Make sure the equation numbers are correct as well both in numbering equation as well as intext mention.

ADCIRC is a finite-element hydrodynamic model that is commonly used for coastal surge assessments [citation]. ADCIRC computes water levels ζ and currents U and V on an unstructured, triangular mesh by applying a linear Lagrange interpolation and the output is a time-series surge hydrographs  obtained through solving the Generalized Wave Continuity Equation (GWCE)

where H=ζ+h is the total water depth; ζ is the deviation of the water surface from the mean; h is the bathymetric depth; U and V are depth integrated currents in the x- and y-directions, respectively; and  are the composite horizontal flux functions that quantify transport of momentum per unit width x- and y-directions, respectively; τ0 is a numerical parameter that optimizes the phase propagation properties (citation).

In this study, the bathymetric data are collected from the official report published by the Nankai Trough Mega-Earthquake Model Study group [citation], and were further interpolated into the mesh to reproduce the seabed topography. The statistical downscaled and bias corrected meteorological forcing data as derived from Eqs. 8 & 9 are utilized as an forcing input. The detailed description of the procedure for numerical modeling using ADCIRC is beyond the scope of this study and can be found elsewhere [3.1.3].

## 2.4 Extraction of process informed covariates with variance decomposition

Instruction for writing the 2.4 subsection: The method described in this subsection as highlighted in “yellow” are used in this study. So you should just make sure the statement are upto the required standard of academic writing, also the information aligns with the overall methodological framework as well as connect with the next. You can refer to citation “3.1.1 and 3.1.3” if you need further methodological clarity before deciding. You can add other references where “citation” is mentioned based on the existing academic papers. Please check the mathematical equation, symbols, as a researcher does it make sense and is it mathematically correct and does it describe the context mathmetically correctly? Also make sure the symbol used here to represent any variable or condition if later repeated in other section then it should be consistent but if make sure same symbol for describing two different variable or condition should have their unique but clear representation and not overlapping the same one. If needed refer to similar existing scientific paper from the internet as well as content of this paper to understand how equation should be written in standard form and specific to this study, as well as if the context should be expanded or reduced. Make this a proper methodological section, establish the relevance and need of this step briefly, then explain what are we doing contextually and mathematically, how this step contributes to the overall methodological framework and finally how this step will be connect to next one or next relevant one. Add more citation only if you feel its necessary. The citation mentioned in number and their corresponding file are uploaded in github, which you can refer for clarity of this method/step.  Make sure the equation numbers are correct as well both in numbering equation as well as intext mention.

Coastal surge extremes are influenced not only by long-term thermodynamic warming, but also by low-frequency internal climate variability that modulates wind fields, regional pressure  gradients and surge-generating meteorological conditions [citation]. These physical mechanisms are often expressed through sea surface temperature (SST) anomaly pattern. Mathematically, the SST anomaly field  is defined as:

where  denote the SST field at spatial location  in the selected study domain  and time t is defined as a date vector, expressed as t = {tyear, tmonth, tday}; is the climatological mean over a reference study period.

Since these patterns are governed by nonlinear ocean-atmospheric feedbacks and high-dimensionality, empirical orthogonal functions (EOFs) are adopted to provide a reduced-order representation of these evolving climate state in a few modes of variability [3.1.1]. The EOF decomposition represents the anomaly field as a finite sum of spatial loading patterns and corresponding time-dependent variance amplitude:

where  is the corresponding  principal component (expansion functions of time); is the k-th EOF spatial loading pattern (an optimal set of basis functions of space); k = 1,…, K is the number of retained modes which are selected based on the combined criteria of maximum variance explanation [3.1.1] and distinguishability based on eigenvalue separation [3.1.3].  the modes are defined:  as the corresponding  principal component (expansion functions of time) and as the k-th EOF spatial loading pattern (an optimal set of basis functions of space). Then, rotated principal components  representing the temporal evolution of the corresponding SST anomaly pattern are retained as a process-informed covariates based on three criteria: statistical robustness, physical interpretability. Statistical robustness is evaluated with explained variance. Physical interpretability is examined based on comparative evaluation of determined climatic modes and its resemblances to known canonical climatic indices like ENSO, PDO or similar. The final outputs obtained based on the EOF method are utilized as time-varying functions to model the coastal extremes as explained in Section 2.5.

2.5 Estimation of nonstationary hazard using BAMLSS framework

Instruction for writing the 2.5 subsection: This section is the MOST CRUCIAL methodological part of the whole framework as it shows how the climate covariates are incorporated in a BAMLSS framework to determine the nonstationary coastal hazard, their exceedance curves which are later used for conditional fragility analysis as well as determining the time-dependent failure probability analysis. The method described in this subsection as highlighted in “yellow” are used in this study. Please refer to citation “3.1.5 and 3.1.6” if you need further methodological clarity before deciding. Add context based on this citation papers and make sure the writing style and information flow is consistent with other sections. You can add other references where “citation” is mentioned based on the existing academic papers. Please check the mathematical equation, symbols, as a researcher does it make sense and is it mathematically correct and does it describe the context mathmetically correctly? Also make sure the symbol used here to represent any variable or condition if later repeated in other section then it should be consistent but if make sure same symbol for describing two different variable or condition should have their unique but clear representation and not overlapping the same one. If needed refer to similar existing scientific paper from the internet as well as content of this paper to understand how equation should be written in standard form and specific to this study, as well as if the context should be expanded or reduced. Make this a proper methodological section, establish the relevance and need of this step briefly, then explain what are we doing contextually and mathematically, how this step contributes to the overall methodological framework and finally how this step will be connect to next one or next relevant one. Add more citation only if you feel its necessary. The citation mentioned in number and their corresponding file are uploaded in github, which you can refer for clarity of this method/step. Make sure the equation numbers are correct as well both in numbering equation as well as intext mention.

MAKE SURE THIS SECTION WILL CONTEXTUALLY, METHODOLOGICALLY AND MATHEMATICALLY EXPLAIN HOW BAMLSS WILL Answer: which climate covariates causes failure probability growth? Is the increase due to location change? Is it due to scale change? Which climate state produces highest failure probability? How much reliability is lost if climate covariates are ignored?

Also I have attached one methodological flowchart as image which you should take as a reference on how the flowchart should look and then use the algorithm used from the paper [3.1.5] and as described contextually and highlighted in “yellow” make similar flowchart for my study. First understand how that flowchart is explaining the method for that study and then go through my study and make similar flowchart but now with information related to my study, I want you to flow the style and structure and information writing style such as equation, symbols are similar to the reference image.  Make sure the looping, sequence are explained similar to the image conceptually and make it scientific. Finally, the context in this topic highlighted in “blue” is just for your reference of writing style and not exactly the information.

General BAMLSS distributional model [3.1.5 and 3.1.6]

The paper defines the general BAMLSS model as:

Additive predictor equation

For this study:

Model term equation: basis function terms

Log-likelihood equation

Log-posterior equation

Prior equation

Updating equation

Multivariate normal prior kernel

The smooth temporal and seasonal effects are regularized through Gaussian prior kernels on spline coefficients, where the smoothing variance controls the degree of wiggliness

Inverse-gamma hyperprior

Small a,b values give approximately flat priors for

b=0, a= -1 gives a flat prior for

inverse-gamma can be sensitive near zero

half-Cauchy priors may be preferable because they have nonzero density at

bamlss posterior structure

= penalty matrix

= smoothing variance

small = smoother function

large = more flexible function

Posterior mode definition

For this study:  the update is based on IWLS backfitting

IWLS backfitting update

With working observations:

= weight matrix from likelihood curvature

= score vector

= prior penalty matrix

= working response

= predictor excluding term j

The code uses BIC for smoothing selection. Conceptually:

Where  is the effective mode complexity

Posterior mean equation:

Since closed form equation is complex, MCMC is needed

Posterior predictive distribution

These are saved as posterior parameter time series

GEV CDF

For this study:

Valid when

Posterior sample of monthly exceedance probability

Posterior mean monthly exceedance probability:

The credible interval is:  which is the time-dependent monthly hazard exceedance curve

Annual exceedance probability from monthly maxima . This is the probability that at least one monthly maximum in year y exceeds level z.

Posterior mean AEP:

The credible interval is: . This gives the time-dependent annual hazard exceedance curve

Posterior hazard curve

The code defines periods such as: early, middle, recent. This allows comparison of hazard changes across early, middle, and recent climate periods.

Annual exceedance probability p:

Return level is:

Return period:

Model diagnostics : model summaries, information criteria, trace plots, autocorrelation plots, effective sample size, quantile residuals, shape-parameter stability check,

Methodological framework for nonstationary extreme-value analysis using BAMLSS

This study models nonstationary coastal storm-surge extremes using a Bayesian Additive Model for Location, Scale, and Shape (BAMLSS) with a Generalized Extreme Value (GEV) response distribution. The methodological basis follows the BAMLSS concept in which not only the conditional mean, but all parameters of a response distribution can be expressed as additive predictors of covariates. In the BAMLSS formulation, a response variable is assumed to follow a parametric distribution whose parameters are linked to predictors through link functions, and each distributional parameter may depend on linear, nonlinear, smooth, random, or spatial effects. This is exactly the conceptual basis required for nonstationary extreme-value analysis, because the storm-surge distribution is allowed to evolve with time, seasonality, and climate-mode covariates rather than being fixed over the full analysis period.

In the R program, the response variable is the monthly maximum surge height. The use of monthly maxima is methodologically important because it preserves the block-maxima logic required by the GEV distribution while providing more observations than annual maxima. The program accepts either sub-monthly surge data, which are internally aggregated to monthly maxima, or data already prepared at monthly resolution. The key input variables are date, surge height, and process-informed covariates such as ENSO1, ENSO2, IPO, and IOD. The surge values are converted to metres if required, filtered to the analysis window, and missing values are removed before model fitting. Covariates are standardized before entering the model so that their coefficients and smooth effects are numerically stable and comparable.

The extreme-value model used in the program is a GEV distribution,

Yt∼GEV(μt,σt,ξt),

where Ytis the monthly maximum surge height at month t, μtis the location parameter, σtis the scale parameter, and ξtis the shape parameter. In physical terms, μtrepresents the central level of monthly extreme surge, σtrepresents the variability or spread of extremes, and ξtcontrols the tail heaviness. The program’s central nonstationary assumption is that μtand σtmay vary with time, month, and climate covariates, while ξis kept constant in the main model because the tail parameter is difficult to estimate reliably with limited block-maxima data. An optional sensitivity model allows ξto vary with time, but this is not the default because nonstationary shape estimation can produce unstable and physically implausible credible intervals.

The main BAMLSS model therefore has the following structure:

μt=fμ,1(timet)+fμ,2(montht)+βμ,1ENSO1t+βμ,2ENSO2t+βμ,3IPOt+βμ,4IODt,
log⁡(σt)=fσ,1(timet)+fσ,2(montht)+βσ,1ENSO1t+βσ,2ENSO2t+βσ,3IPOt+βσ,4IODt,
ξt=ξ.

Here, fμ,1and fσ,1are smooth time effects, while fμ,2and fσ,2are cyclic seasonal smooths for the month of the year. This means the program does not force the trend to be linear. Instead, it lets the data estimate smooth nonlinear evolution in the surge distribution. This directly reflects the BAMLSS idea that each distributional parameter can be linked to flexible additive predictors built from model terms such as splines and covariates.

The program fits several nested BAMLSS-GEV models. Model 0 is stationary, with constant μ, σ, and ξ. Model 1 allows only μto vary with time, seasonality, and covariates. Model 2 allows both μand σto vary and is treated as the primary nonstationary BAMLSS model. Model 3, if requested, additionally allows a simple time effect in ξ. This nested structure is methodologically useful because it separates three questions: whether the surge level is changing, whether the variability of extremes is also changing, and whether the tail shape needs to be time-dependent.

The estimation algorithm follows the two-stage BAMLSS logic described by Umlauf et al. First, the program obtains posterior mode estimates using opt_bfit, which is based on iteratively weighted least squares and backfitting. This gives stable starting values for a complex additive distributional model. Second, posterior uncertainty is sampled using sam_GMCMC, the general MCMC sampler in bamlss. This two-stage strategy mirrors the BAMLSS framework, where posterior mode estimation can be used first, followed by MCMC sampling for full Bayesian inference.

The important point is that the program does not only estimate one “best” curve for μt, σt, and ξt. It generates posterior samples of these parameters. For each month and each posterior draw, the fitted model provides a possible GEV distribution. This allows uncertainty in the model parameters to be carried forward into hazard exceedance probabilities and return levels. This is consistent with the BAMLSS paper’s argument that fully Bayesian inference is useful because posterior samples allow inference for complex functions of the model, not only for individual coefficients.

After fitting, the program predicts posterior parameter trajectories:

μts,σts,ξts,

where sdenotes an MCMC posterior draw. These posterior trajectories are then inserted into the GEV cumulative distribution function,

Fts(z)=exp⁡1ξtsz-μtsσts-1/ξts,

with the Gumbel limit used when ξis close to zero. The monthly exceedance probability for a surge level zis then computed as

Pts(Y>z)=1-Fts(z).

The program averages these exceedance probabilities across posterior draws to obtain posterior mean exceedance probabilities and credible intervals. This is a critical methodological point: the exceedance probability is computed from the posterior predictive GEV distribution, not from the posterior distribution of μ, σ, or ξalone. In simple terms, the program first builds the full distribution of future monthly maxima and then asks, “How often does this distribution exceed a chosen level?” That is the correct hazard calculation.

The program then converts monthly exceedance probabilities into annual exceedance probabilities. For a given year y, the annual exceedance probability is computed as

AEPys(z)=1-m=112Fy,ms(z).

This means the annual probability is the probability that at least one month in that year exceeds surge level z. This step is important because engineers usually interpret hazard in annual terms, while the statistical model is fitted to monthly maxima. The program therefore preserves the richer monthly structure while still producing annual hazard curves, annual exceedance probabilities, and return-level curves that are meaningful for engineering risk and reliability analysis.

The program also performs model diagnostics. It writes model summaries, information criteria, trace plots, autocorrelation plots, effective sample size tables, and quantile residual diagnostics. It also warns if the posterior interval of ξbecomes too wide, because this would indicate poor tail identification. This is methodologically important: for extreme-value analysis, a visually attractive nonstationary curve is not enough; the MCMC chain must mix reasonably, the posterior uncertainty must be interpretable, and the tail behaviour must remain physically plausible.

Algorithmic workflow implemented in BAMLSS_surge_analysis.R

The program can be described as the following algorithm.

Step 1: Data preparation. The script reads the surge and covariate CSV file, parses dates, filters the data to the target period, converts surge units if necessary, removes incomplete rows, and aggregates sub-monthly records to monthly maxima if the input is not already monthly. It then creates year, month, sequential time index, standardized time, and standardized covariates.

Step 2: BAMLSS-GEV model specification. The script defines a GEV distributional regression model where the location, scale, and shape parameters are represented by separate formulas. The primary model allows smooth time effects, cyclic seasonal month effects, and climate covariates in μand σ, while keeping ξconstant.

Step 3: Posterior mode estimation. The model is first fitted using opt_bfit. This is the computational stabilisation step. In analogy, it is like first finding the valley floor before starting a detailed probabilistic exploration of the terrain.

Step 4: MCMC sampling. The posterior mode estimates are used as starting values for sam_GMCMC. This produces posterior samples of all model terms and parameters, allowing uncertainty propagation into hazard curves.

Step 5: Posterior prediction. The fitted model is used to predict posterior samples of μt, σt, and ξtfor each month.

Step 6: Hazard exceedance calculation. For each surge threshold z, each month t, and each posterior draw s, the program computes 1-Fts(z). These are summarized as posterior mean, median, and credible intervals.

Step 7: Annualization. Monthly CDFs are multiplied within each year to compute annual exceedance probability:

AEPy(z)=1-m=112Fy,m(z).

Step 8: Return-level estimation. The program inverts annual exceedance curves to estimate return levels for target AEP values such as 0.50, 0.10, and 0.01, corresponding approximately to 2-year, 10-year, and 100-year return levels.

Step 9: Diagnostic and output generation. The program exports processed data, fitted model objects, diagnostic summaries, posterior parameter time series, monthly and annual hazard curves, period-wise comparisons, selected-year hazard curves, and return-level curves.

How BAMLSS_benchmark_comparison.R validates and compares the BAMLSS process

The second R program, BAMLSS_benchmark_comparison.R, is designed as an external benchmark and validation layer. It does not assume that BAMLSS is automatically superior. Instead, it compares BAMLSS against simpler and alternative nonstationary GEV approaches on a common data set and common hazard grid. The script explicitly states that BAMLSS is treated as the flexible distributional regression reference, but “we do not assume it wins; we test it.”

The benchmark script fits several alternative GEV models. First, it fits a stationary GEV model, which assumes no trend and no covariate-driven change. This is the baseline: any nonstationary model must improve upon it to justify its added complexity. Second, it fits trend-based GEV models, including a linear time trend in μ, a model with time-varying μand σ, and a harmonic seasonal model using sine and cosine seasonal terms. These models test whether simple parametric nonstationarity can explain the data without the flexibility of splines. Third, it fits moving-statistic GEV models, where moving averages and moving standard deviations are used as covariates inside a GEV model. Fourth, it fits rolling-window stationary GEV models, where a stationary GEV is repeatedly fitted within trailing windows such as 10, 20, or 30 years. The benchmark script treats rolling-window GEV as a robustness check because each window uses fewer observations and is sensitive to window length.

The comparison is not based only on AIC or BIC. The script prioritizes predictive and diagnostic criteria: out-of-sample log score, PIT diagnostics, quantile residuals, return-level stability, hazard-curve stability, physical plausibility, and sensitivity tests. AIC and BIC are reported only as supporting metrics because comparing Bayesian and frequentist information criteria across different model classes is imperfect.

For BAMLSS, the benchmark script loads the outputs produced by BAMLSS_surge_analysis.R, including the primary Model2_ns_mu_sigma_mcmc.rds, posterior parameter time series, monthly and annual hazard curves, selected-year hazard curves, period-wise hazard curves, return levels, and information criteria. If these files are missing, the script logs the missing outputs and continues in benchmark-only mode.

For predictive validation, the benchmark script computes the posterior-predictive log score for BAMLSS using the log-sum-exp method. This is important because BAMLSS produces many posterior draws, so the predictive density is a mixture over posterior samples rather than a single fitted density. For each observation, the script evaluates the GEV log density under each posterior draw and then averages on the log scale using log-sum-exp for numerical stability.

For the frequentist benchmark models, uncertainty is propagated by drawing parameters from an approximate multivariate normal distribution,

θb∼MVN(θ,V),

or by parametric bootstrap. These parameter draws are then passed through the same GEV CDF used for BAMLSS. This makes the comparison fair because all models are evaluated through exceedance probabilities and return levels, not simply through fitted parameter values.

In summary, the benchmark script compares BAMLSS against four main nonstationary extreme-value alternatives:

- Stationary GEV — no time or covariate effect; baseline model.

- Trend-based GEV — linear time trend and/or harmonic seasonal terms in μ, optionally σ.

- Moving-statistic GEV — moving averages or moving standard deviations used as covariates inside the GEV model.

- Rolling-window GEV — stationary GEV fitted repeatedly within moving time windows.

The comparison asks whether the flexible BAMLSS model gives better predictive performance, better calibrated PIT values, more stable return levels, more physically plausible hazard curves, and acceptable uncertainty compared with these simpler models.

Methodological interpretation

The overall methodology is strong because it follows a logical hierarchy. The first program estimates a flexible Bayesian nonstationary GEV model in which the full distribution of surge extremes evolves with time, seasonality, and climate covariates. The second program checks whether this flexibility is justified by comparing BAMLSS against simpler stationary, parametric trend, moving-statistic, and rolling-window GEV models.

The main scientific claim supported by this workflow is not simply “BAMLSS fits the data.” The stronger and more defensible claim is:

A Bayesian distributional GEV model was used to estimate time-varying storm-surge hazard by allowing the location and scale of monthly maxima to evolve as smooth functions of time, seasonality, and climate-mode covariates. Posterior uncertainty was propagated through the GEV distribution to obtain monthly and annual exceedance probabilities, return levels, and credible intervals. The resulting hazard estimates were evaluated against stationary, trend-based, moving-statistic, and rolling-window GEV benchmarks using predictive scores, residual diagnostics, return-level stability, hazard-curve stability, and physical plausibility.

That wording is methodologically safer than claiming BAMLSS is automatically the best method. It shows that BAMLSS is the main proposed model, while the benchmark program provides independent evidence for whether its added complexity is justified.

The excess water level above the river cross-sectional height (i.e., flood depth Hexc), as indicated in Fig. 2, is used as the primary variable for embankment design. Flooding events are often modeled with the Poisson process [86,87]. Due to climate change effects, the intensity and frequency of maximum flood depths become a non-stationary Poisson process. In addition, the excess of water Hexc above the river height hr can be approximated using the POT method [88]. A combination of the extreme value analysis using the POT method and the non-homogeneous Poisson process is developed herein to consider climate change effects.

In the first step, the intensity of the river water level exceeding the river height Hexc and the corresponding arrival times of these events are recorded based on the time-variant river water level from the hydraulic assessment, as illustrated in Fig. 3. From the statistics of the inter-arrival time between flooding events, the mean recurrence rate λ can be estimated. Let the inter-arrival and arrival times of the flooding events be denoted as W and S, respectively. For simplicity, the mean and standard deviation associated with the random variable Hexc and the mean recurrence rate of the flood events λ are assumed to increase linearly with time t, such that:

where μHexc (t) and σHexc (t) are the time-variant mean and standard deviation of the flood depth Hexc (m), respectively, λ (t) is the time-variant mean recurrence rate of the flooding events (event per unit of time), μHexc,0 and σHexc,0 are the mean and standard deviation of the flood depth at present time t0, respectively, λ0 is the mean recurrence rate of the flooding events at present time t0, mHexc,μ and mHexc,σ  are the rate of change per amount of time t associated with the mean and standard deviation of the flood depth Hexc (meter per amount of time), respectively, and mλ is the rate of change per amount of time t for the mean recurrence rate λ. These time-variant parameters are statistically deduced from the output of hydraulic assessment illustrated in Fig. 3 using time-series analysis such as moving average (MA) and moving standard deviation (MSD) methods [89,90].

The Generalized Pareto (GP) distribution is often utilized for modeling exceedances due to its adaptability in capturing the tail behavior of the distribution [91–93]. The CDF of the GP distribution and the corresponding statistical parameters can be expressed as follows:

where FHexc (hexc) describes the probability of Hexc having a value less than hexc, α and ν are the shape and scale parameters associated with the GP distribution, respectively. Since the mean and standard deviation of flood depth are time-dependent, as described in Eqs. (12) to (13), the time-variant statistical parameters associated with the GP distribution can be evaluated by rearranging Eqs. (16) and (17) as follows:

According to Eqs. (15) to (19), the GP distribution becomes time-dependent due to the non-stationary climate change effects. Assuming that the flood events follow a non-homogeneous Poisson process, the CDF of the maximum flood depth can be evaluated as [94]:

where FHexc,max(tD) (hexc) and fHexc,max(tD) (hexc) are the CDF and the probability density function (PDF) of maximum flood depth from present time t0 to an analyzed time tD, respectively, and FHexc(s) (hexc) is the time-variant GP distribution with statistical parameters according to Eqs. (18) and (19).

# 3. Time-dependent reliability assessment framework under the process-informed climate states and nonstationary extreme hazards.

3.1 Fragility assessment of structures subjected to coastal hazard

Instruction for writing the 3.1 subsection: This methodological step should be written based on thw information provided in the Microsoft word document “fragility.docx” and [3.1.7] both of which are uploaded in github. This study actually use statistical/empirical fragility function based on the analysis done in paper [3.1.7] so it should be written accordingly and the writing style and amount of information cn be as per the template highlighted in “blue” in this section. But make sure here it is mentioned that it is adopted from that study but the mathematically and contextual description should be for general methodological framework for my study and not present it as case study. I want to specify all the case details in the “illustrative example” section of this manuscript. So provide necessary context, tables or fragility figure if required here. However, a little but most important detail you need to add is that for this study since we are looking for the surge flooding on structure so we will be looking at the serviceability condition, that is why we considered exceedance moderate damage state as failure or our limit state for failure assessment. So you should just make sure the statement are upto the required standard of academic writing, also the information aligns with the overall methodological framework as well as connect with the next. You can add other references where “citation” is mentioned based on the existing academic papers. Please check the mathematical equation, symbols, as a researcher does it make sense and is it mathematically correct and does it describe the context mathmetically correctly? Also make sure the symbol used here to represent any variable or condition if later repeated in other section then it should be consistent but if make sure same symbol for describing two different variable or condition should have their unique but clear representation and not overlapping the same one. If needed refer to similar existing scientific paper from the internet as well as content of this paper to understand how equation should be written in standard form and specific to this study, as well as if the context should be expanded or reduced. Make this a proper methodological section, establish the relevance and need of this step briefly, then explain what are we doing contextually and mathematically, how this step contributes to the overall methodological framework and finally how this step will be connect to next one or next relevant one. Add more citation only if you feel its necessary. The citation mentioned in number and their corresponding file are uploaded in github, which you can refer for clarity of this method/step.  Make sure the equation numbers are correct as well both in numbering equation as well as intext mention.

A reliable assessment of economic loss depends to a large extent on the accuracy of the vulnerability information of structures. Empirical fragility curves can be used to establish realistic postearthquake event conditions. In this study, the tsunami fragility curves for two structural typologies (i.e., wooden and nonwooden structures) are adopted from De Risi et al. [48] based on damage data from the 2011 Great East Japan earthquake. According to Suppasri et al. [49], uncertainties associated with tsunami vulnerability assessment can be reduced by taking into account the effects of flow velocity, distance from the coastline, and building orientation to the sea on tsunami intensity, although this is outside the scope of the present study. To simplify the discussion of the life-cycle risk assessment under nonstationary sea-level rise hazards, it is assumed that the damage probability of the structure is estimated only in terms of the inundation depth and structural typology. It is noteworthy that the effects of ground motion-induced damage are considered in the tsunami fragility models [48] and have been commonly used in previous studies of tsunami risk assessment in Japan [7, 25].

Three damage states are adopted herein: none, moderate, and complete, denoted as ds1, ds2, and ds3, respectively. By approximating the fragility curve as a lognormal distribution, the probability of a building jb with structural type st (hereafter denoted as jb (st)) having a damage state dsq (q = 1, 2, and 3 correspond to none, moderate, and complete damage, respectively) given that the tsunami intensity H in grid cell k equals h (i.e., Pjb(st) (DS = dsq | Hk = h)) can be described as follows:

where Φ(∙) denotes the CDF of the standard normal distribution and νt,q (st) and σt,q (st) are the mean and standard deviation of the tsunami fragility curve associated with damage state q and structural type st, respectively, with the values given in Figure 2; st = 1 and st = 2 represent wooden and nonwooden structures, respectively.

The building tsunami fragility curves, including the statistical parameters associated with Equation (4) for each structure type and damage state, are shown in Figure 2 [48]. These fragility curves are used to evaluate the time-dependent reliability and life-cycle risk of building structures subjected to tsunamis under non-stationary sea-level rise.

Figure 2. Tsunami fragility curve for building structures (adapted from [48])

3.2 Time-dependent failure probability of structures

Instruction for writing the 3.2 subsection: For this step I want you to write based on two section:

- You need to search on this internet find relevant Q1 journal paper and academic article that gives general mathematical foundation for this step and based on the context highlighted in “yellow” for this section prepare me the context, method and math of this section. Give proper citation to the paper if you reference any. You can use the context highlighted in “blue” as a reference for writing style and structure. Also make sure the statement are upto the required standard of academic writing, also the information aligns with the overall methodological framework as well as connect with the next. Please check the mathematical equation, symbols, as a researcher does it make sense and is it mathematically correct and does it describe the context mathmetically correctly? Also make sure the symbol used here to represent any variable or condition if later repeated in other section then it should be consistent but if make sure same symbol for describing two different variable or condition should have their unique but clear representation and not overlapping the same one. If needed refer to similar existing scientific paper from the internet as well as content of this paper to understand how equation should be written in standard form and specific to this study, as well as if the context should be expanded or reduced. Make this a proper methodological section, establish the relevance and need of this step briefly, then explain what are we doing contextually and mathematically, how this step contributes to the overall methodological framework and finally how this step will be connect to next one or next relevant one. Make sure the equation numbers are correct as well both in numbering equation as well as intext mention.

- Next I want you to prepare me a Monte Carlo simulation pseudoalgorithm for my study from start until finish. Take the statement, procedural writing style, where mathmetical symbol and equation were used exactly as reference in “blue” highlighted statement in this section for your writing style reference. I want the pesudoalgorithm to look scientific and like you are reading a computational step by step with proper math symbol yet with clear and brief explanation.

You start from:

Ht∼GEV(μt,σt,ξ)

but instead of directly calculating annual failure probability, you explicitly integrate the fragility function:

Pf(t)=∫P(F∣h)fH(h,t)dh

Then:

PLC=1-t=1T(1-Pf(t))

You care about how the evolving hazard distribution interacts with fragility. Reliability is the result of that interaction.

If your framework can quantify:

Covariate→(μ,σ)→HazardChanges→ReliabilityChanges

and decompose the reliability loss into identifiable climate-process drivers,

then it becomes substantially more interesting. This paper does not merely estimate future failure probability; it explains the mechanism by which climate-process nonstationarity propagates into structural reliability

Minimum metrics you should derive

For your paper, I would recommend these as the core set:

- μ(t), σ(t)from BAMLSS

- Time-varying exceedance probability P(Ht>hc)

- Time-varying return level zT(t)

- Annual failure probability Pf(t)

- Life-cycle failure probability PLC

- Expected number of exceedances/failures

- Design-life level

- Covariate contribution to Pf(t)

10. Covariate sensitivity / contribution metrics

This is very important for novelty.

You should quantify:

∂μ∂Xi,∂σ∂Xi

and then:

∂Pf∂Xi

This tells:

Which climate covariate most increases failure probability?

For example:

- ENSO mainly changes μ

- IPO mainly changes σ

- sea-level rise shifts μ

- combined ENSO + SLR produces highest Pf(t)

This is where your framework becomes more than just a curve.

So derive a decomposition like:

| Source | Effect on hazard | Effect on failure probability |
| --- | --- | --- |
| ENSO | changes μ | increases/decreases annual Pf |
| IPO | changes σ | changes tail spread |
| SLR | shifts μupward | raises baseline Pf |
| ENSO + SLR | combined high state | produces reliability-critical years |

This will support the claim that your research gives process-informed reliability understanding, not only nonstationary hazard estimation.

Each fragility curve depends on the structural type. The tsunami hazard curve enclosed by inundated grid cell k is assumed to be uniform. This results in the same failure probability for structures with the same structural type st located in the same grid cell k. The annual failure probability of any structure jb (st) located in inundated grid cell k without considering the effects of sea-level rise can be estimated by convolving the tsunami hazard with the fragility curve according to Equation (4), as follows:

where Pjb(st), k (DS = dsq) is the annual failure probability of building jb (st) with damage state dsq located in inundated grid cell k and FHk(h) is the CDF of the tsunami hazard in grid cell k containing building structure jb (st).

Due to the effects of non-stationary sea-level rise, the annual failure probability is no longer constant. Therefore, the time-dependent failure probability of structure jb (st) in grid cell k considering sea-level rise effects at a particular time t, Pjb(st), k (DS = dsq, T = t), can be determined by using Equations (3) and (5) as follows:

Based on the time-dependent failure probability described above, the life-cycle risk under non-stationary sea-level rise effects is developed according to a compound renewal process considering the uncertainties associated with the number of earthquake occurrences and the corresponding arrival times.

To determine the partial factor considering climate change effects, the embankment height should be designed with sufficient reliability to prevent overtopping and slope instability failure. However, due to the intricate expression in Eqs. (23) to (26), it becomes increasingly complex to employ the FORM method since it necessitates the transformation to the standard normal space and involves derivating the limit state function with respect to each random variable. A more suitable way is to apply the ellipsoid approach introduced by Low & Tang [105]. This approach aims to find the minimum ellipsoid that touches the failure surface, as illustrated in Fig. 5, by satisfying the following constrained nonlinear optimization problem:

subject to:

where βH is the Hasofer-Lind reliability index without climate change effects, as indicated in the red line in Fig. 5, X* is the set of MPP of the corresponding failure mode, μXN and σXN are the set of normalized mean and standard deviation, respectively, ϕn and Φn are the PDF and CDF of standard normal distribution, respectively, and r is the correlation matrix. Since the limit state function in Eqs. (23) to (26) is significantly complex due to the dependence of the phreatic line geometry on the maximum flood depth Hexc,max, the sequential quadratic programming (SQP) method is adopted to solve the nonlinear constrained optimization in Eqs. (27) to (30). The details of the SQP algorithm are described thoroughly in Boggs & Tolle [106].

To achieve a predetermined level of safety, the design point of each variable is evaluated based on the target reliability βt with βt < βH. In addition, to assess the value of the design point XD*, it is necessary to obtain the original reliability index βH and the corresponding MPP X* by solving Eqs. (27) to (30). It should be noted that the limit state functions in Eqs (23) to (26) associated with the constraint in Eq. (28) are dependent on the embankment height he, which is designed according to Eq. (22) and reliability target βt. The iterative procedure to estimate the design point XD* is shown in Fig. 6 and described as follows:

- Specify the target reliability index βt and the tolerable error ϵ

- Determine the embankment design height he according to the limit state function in Eq. (22) and target reliability index βt.

- Evaluate the reliability index βH and MPP point X* by solving Eqs. (27) to (30).

- In the first iteration (i.e., j = 1), the evaluated MPP point X* is defined as .

- The normalized mean  and standard deviation  for the j-th iteration are evaluated based on   as follows:

- The reliability index βj,old for the j-th iteration is assessed according to the following expression:

- A reduction factor of ψj = (βt / βj,old) is introduced and employed to estimate the new MPP  and the associated normalized mean  and standard deviation  as follows:

- A new reliability index βj,new is evaluated in the same manner as Eq. (33):

If the condition |βt – βj,new|2 < ϵ is satisfied, with ϵ as the predefined tolerable error,  is assigned as the design point XD*. Otherwise, proceed to the next iteration (i.e., j = j+1) and repeat steps (2) to (5) with  =  until the condition |βt – βj,new|2 < ϵ is satisfied.

The LRFD framework introduced in this section offers a versatile approach that extends its applicability beyond embankment cases, such as bridges under scouring hazards, roof structures against wind and rain loads, and seawalls to mitigate sea-level rise. Given the non-stationary structural deterioration and increasing load actions in the future, this framework can be considered for adoption in future to address the challenges posed by climate change. To demonstrate the effectiveness of the proposed framework, illustrative examples are presented in the following section.

Monte Carlo simulation pseudo-algorithm

Therefore, an MCS procedure is proposed herein to evaluate the risk curve associated with cumulative loss until tD (1 – FY(tD) (y)) considering the effects of nonstationary sea-level rise. The MCS procedure illustrated in Figure 4 is implemented in the following steps.

- Prior to MCS, the analyzed region is divided into grid cells, and preliminary assessments associated with the building data (i.e., structural type, location, floor area of each structure, and structure fragility curve) and earthquake occurrence modeling (Poisson or non-Poisson process interarrival time, fW (t)) are performed. In addition, the present time t0, time period Tp for life-cycle risk assessment (i.e., tD = t0 + Tp), and the number of MCS trials NMCS are determined.

- Time-variant sea-level rise hazards over the analyzed fault and conditional tsunami hazards for each building structure are assessed according to Equation (2) and tsunami propagation analyses, respectively.

- The MCS process is initiated. A random variable tR is sampled to simulate the time of first earthquake occurrence through the inverse transform sampling of fW (t | T > t0) relative to t0 according to Equation (8).

- If the variable tn+1 (i.e., tn+1 = t0 + tR) is less than the analyzed time tD, the first earthquake is registered. Otherwise, the earthquake is assumed not to occur and proceeds to the subsequent MCS trial.

- If an earthquake occurs, the number of earthquake occurrences is updated (i.e., n = n + 1), and the sea-level rise CDF FΨ (ψ(xr, tn)) at time tn is recorded based on the time-variant sea-level rise hazard curves.

- According to Equation (3), the tsunami hazard curve is estimated at time tn by convolving the conditional tsunami hazard curves with the sea-level rise hazard at time tn.

- Based on the tsunami hazard curve at time tn, the inundation depth in each inundated grid cell k hi,n = {h,i,n,1, h,i,n,2, …, h,i,n,k, …, h,i,n,nk}T is sampled.

- The probability associated with complete and moderate damage states, denoted as points X1 and X2 in Figure 4, respectively, is estimated for each building structure jb (st) located in the corresponding grid cell k based on the inundation depth h,i,n,nk generated according to Equation (4). Moreover, random numbers, denoted as RN in Figure 4, are generated according to a uniform distribution for each building structure jb (st) located in the corresponding grid cell k.

- The damage state of each building structure jb (st) in grid cell k, DSi,n = {dsi,n,k,1(st), dsi,n,k,2(st), …, dsi,n,k,njb(st)}T, is assigned as no damage ds1, moderate damage ds2, or complete damage ds3 if RN is greater than X2, RN is between X1 and X2, or RN is less than X1, respectively. Steps viii to ix are repeated for all inundated grid cells k (i.e., for k = 1, 2, …, nk), such that the damage state for all inundated structures in the analyzed region is obtained. The total number of inundated structures in the analyzed region can be estimated as [nk × njb(st = 1)] + [nk × njb(st = 2)].

- The building portfolio loss Li,n at time tn is estimated according to Equation (14) by aggregating the individual building losses based on the structure type, replacement cost, damage ratio, and structure floor area.

- The time tn is considered the last earthquake occurrence. Therefore, tR is generated relative to tn based on fW(t). The variable tn+1 = tn + tR is compared with tD. If tn+1 is less than tD, the second earthquake is recorded, and the simulation proceeds to the next step.

- Steps iv to xi are repeated until the updated time of the most recent earthquake events exceeds the analyzed time tD. After the updated time exceeds tD, the cumulative loss for MCS trial i is calculated by summing all of the sampled losses (i.e., Yi = Li,1 + Li,2 + … + Li,n), and a new MCS trial begins (i.e., i = i + 1).

- Steps ii to xii are repeated until the number of MCS trials exceeds the predefined number of simulations NMCS. The risk curve associated with cumulative loss is evaluated until tD (1 – FY(tD) (y)), as shown in Equation (15), based on the occurrence frequency of the generated cumulative losses (i.e., Y1, Y2, …, YNMCS).

The proposed framework is not limited to tsunamis under non-stationary sea-level rise effects but can also be implemented for various life-cycle risk assessment cases with multihazard exposure in the context of climate change, such as cases with earthquakes and scouring hazards due to extreme precipitation. To the best of the authors’ knowledge, no studies have considered the effects of non-stationary sea-level rise in life-cycle risk assessments of infrastructure assets subjected to tsunamis with non-Poissonian earthquake occurrence. The proposed framework can significantly improve the accuracy of life-cycle risk assessment, which is essential for enhancing disaster preparedness in coastal communities.

Due to the impacts of climate change, numerous studies have revealed that the capacity of structures is rapidly diminishing (e.g., concrete corrosion under climate change [107]) and that both the intensity and frequency of load actions are increasing [108]. As a result, the normalized mean of structural resistance μRN in Fig. 5 is projected to decrease and become , while the normalized load actions μSN increases and becomes  due to the climate change effects. In addition, since climate change is deemed a non-stationary process [46,47], the normalized standard deviation of structural resistance and load actions become and , respectively. These alterations would imply a shift in the center points and enlarge the radius of the equivalent normal 1–σ dispersion ellipsoids, as illustrated in the blue line of Fig. 5.

The overall process of estimating the partial factors for LRFD-based design considering climate change effects is summarised as follows:

- Determine the target reliability index βt, characteristic values of random variables Xk, and the associated PDF and CDF, and embankment height he based on Eq. (22) and βt.

- Estimate the reliability index βH and the associated MPP X* by solving Eqs. (27) to (30).

- When considering climate change effects, the reliability index and the associated MPP are assessed and denoted as βH,cc and Xcc*, respectively, with βH,cc < βH.

- Evaluate the design points XD* according to the iterative procedure in Fig. 6.

- When considering climate change effects, evaluate the design points XD,cc* according to the iterative procedure in Fig. 6.

- Assess the characteristic values associated with structural resistance Rk and load actions Sk according to Eqs. (24) and (25), respectively.

- Similar to step (4), calculate the design points associated with structural resistance RD* and load actions SD* based on XD*.

- Considering climate change effects, calculate the design points for structural resistance RD,cc* and load actions SD,cc* based on XD,cc* in a similar manner as steps (4) and (5).

- The reduction factor γRa and amplification factors γSa to ensure structural safety are evaluated according to Eqs. (2) and (3).

- The reduction factor γRb and amplification factors γSb to consider climate change effects are evaluated according to Eqs. (5) and (6).

As the number of random variables influenced by climate change increases, the partial factors γRb and γSb will progressively deviate from the value of 1.

# 4. Illustrative examples

Instruction for writing the result: results relevant to show the claim made in the introduction and method, so everything highlighted “yellow” in this section are specific information to this study and everything highlighted in “blue” are just for your writing style reference as well as for you to underastand how to write statement in result section and what type of figure should be discussed and how it should be discussed.

Before writing this section again go through introduction and methodology, then for each subsection prepare list of figures, tables or results that should be presented there to justify the method as well as research objective and also write what type of discussion should be made in each sections. You can also provide citation to the similar researches so that I can just make the figure, explain it and add those citation for comparison, but make sure you describe what they found such that I can compare myself. But you need to tell me what type of discussion or information should be made in all of the subsection

## 4.1. Study area setup, numerical setup

- Study done in Tokyo bay, structure is bridge and the classification is based on information provide in citation [3.1.7]

- Figure: study area, mesh or bathymetry map,

Statistical downsaclign time series with and without downscaling, comparing with observed time-series for observed station value, Q-Q plot, monthly comparison

numerical validation of ADCIRC

To validate the reliability of the ADCIRC simulations, historical wind speed and atmospheric pressure data recorded during Typhoon Hagibis (2019) were input into the model. The simulated water levels were compared with tidal gauge observations. Figure The results indicate that ADCIRC reasonably reproduce water level variations prior to typhoon landfall, confirming the accuracy of predicted peak surge timing and maximum water level deviation. Certain discrepancy are observed in the simulated water level recession trends, however, the study is evaluated based on the daily maximum surge deviation, and the primary performance metric is peak water level rather than surge duration [citation]. Thus the impact is considered negligible.

In the 2018 Japan flood, Fukuyama city’s coastal areas suffered significant damage [109]. Therefore, a river embankment near the Ashida River in Fukuyama city, Japan, is designed according to the proposed LRFD framework. The city’s location in Hiroshima Prefecture and the associated topographic information are shown in Fig. 7. Moreover, the spatial distribution of land use and soil types of Fukuyama city are employed to SWAT, as shown in Fig. 8. On the other hand, the hydraulic model for the Ashida River depicted in Fig. 9 is assumed to have a uniform river height (hr = 3.6 m) for simplicity. Based on these data, the hydrological and hydraulic assessments were carried out on a daily basis. However, to provide greater accuracy of the maximum water level height and to reduce the computational demand, the relationship between the daily average and 24-hour maximum river water level is derived from Fukuyama city’s stream station data (Fig. 10(a)). For SWAT input purposes, the maximum and minimum daily temperature are derived from the daily average temperature using linear relationships to eliminate the need for hourly temperature data from each GCM, as shown in Figs. 10(b) and (c).

Fig. 11 shows the verification results of the hydrological and hydraulic model developed in this study based on the observed data from weather and stream stations. According to Fig. 11(a), the SWAT results overestimated the river baseline flow and performed well in predicting the extreme river flow, especially during the 2018 Japan flood. The performance of the SWAT model can be quantified using the Nash-Sutcliffe efficiency (NSE) [110]. The NSE value for the uncalibrated SWAT model is 0.499, implying that the SWAT performance is satisfactory [111]. On the other hand, the hydraulic model performed very well in predicting the river height for both river baseline and extreme conditions, as shown in Fig. 11(b). The relationships provided in Figs. 11(b) and 10(a) are utilized to correct the river height obtained from hydrological and hydraulic analyses considering climate change effects, as discussed in the following subsections.

## 4.2. Process informed covariates

The EOF and principal components determining from the CMIP6 based GCM dataset for 5 GCM  and 2 SSP scenarios (SSP 4.5 for understanding moderate scenarios, SSP585 for understanding extreme scenarios) models and validating with the observed dataset all the process are as per following the methodological step as mentioned in the paper [3.1.4]. Explain why chosen climate indices for this study are relevant for teleconnection studies of Japan based on existing literature from the internet with proper citation.

- Figures: Accuracy between 5 model and ERSST observed data to canonical indices box, region, For one GCM model show the statistical robustness through explain variance or scree plot and discuss statistical robustnessness y, compare spatial maps with canonical indices wither heat map or contour maps and discuss about physical interpretaility.

Prior to hydrological and hydraulic analyses, the rainfall data and temperature for each GCM were statistically downscaled using the QDM method described in Section 3. Daily rainfall and temperature data from 8 GCMs and 4 SSP emission scenarios (i.e., SSP 2.6, 4.5, 6.0, and 8.5) are considered herein, spanning 2021 to 2100. Fig. 12 shows the observed and statistically downscaled precipitation and temperature data from the Fukuyama weather station, specifically for the Canadian GCM CanESM5. Daily temperature is assumed to be a normal distribution [112], while Kernel density estimation (KDE) is adopted for daily precipitation due to the poor performance of the gamma distribution for extreme values. As shown in Fig. 12, the statistical characteristics of precipitation and temperature become identical to that of the corresponding observed data, highlighting the importance of statistical downscaling.

The effects of climate change emission scenarios on precipitation and temperature were assessed by evaluating the mean and standard deviation of all GCMs under different SSPs and are presented in Fig. 13. The annual precipitation trends are relatively similar for SSP 4.5 and 8.5, with higher SSP values indicating more significant variability. Conversely, a substantial increase in temperature was projected under SSP 8.5 compared to SSP 4.5. These observations suggest that river streamflow in the Ashida River is expected to be lower under more severe climate change conditions due to the higher evapotranspiration rate, as indicated in Eq. (11).

Fig. 14 shows the simulated river streamflow evaluated in the Ashida River outlet, as indicated in Fig. 7(c). The streamflow is expected to exhibit greater intensity than the 2018 Japan flood for SSP 2.6 and 4.5, in contrast to SSP 6.0 and 8.5. Additionally, focusing on individual GCMs, the frequency of flooding events that are more severe than the 2018 Japan flood is projected to increase for lower SSP values. Finally, the water level from the riverbed was evaluated based on the hydraulic assessment and is presented in Fig. 15.

## 4.3. Probabilistic assessment of the maximum surge/flood depth using BAMLSS

Discuss use of both covariates and surge for process informed extreme value analysis and make sure the statement are not redundant with the methodology section.

Figures: partial effect plots to show change of covariates on location and scale parameter and what it means, hazard exceedance curves, posterior uncertainty bands

Validation through comparison with other nonstationary and stationary hazard curves and what this means and how ourBAMLS results is significant

discuss what we understand while answering Answer: which climate covariates causes failure probability growth? Is the increase due to location change? Is it due to scale change? Which climate state produces highest failure probability? How much reliability is lost if climate covariates are ignored? From the hazard perspective

To assess the distribution of maximum flood depth considering the non-stationary climate change effects, the statistics associated with the flood depth are evaluated by subtracting the water level results from the hydraulic assessment with the river height hr. Moreover, the time-variant parameters associated with the mean, standard deviation, and mean recurrence rate of flood events are estimated through the MA and MSD approaches based on these statistics. For instance, Fig. 16 presents the flood depth and interarrival time between successive flood events projected by GCM CanESM5 under SSP 4.5 and 8.5. The MA and MSD were evaluated with 20 consecutive data points in each average calculation to obtain a relatively smooth moving average, as shown in Fig. 16. The MA and MSD data are then fitted into linear trends to simplify the time-varying statistical parameters. According to the CanESM5 GCM model, it is projected that the mean and standard deviation for flood depth will be more pronounced under SSP 4.5 compared to SSP 8.5. In contrast, a reduced mean inter-arrival time is observed for SSP 8.5, indicating that, despite the lower flood intensity in SSP 8.5, an upsurge in the frequency of flood events is anticipated in the future. It is noteworthy that the mean recurrence rate λ can be determined as the reciprocal of the inter-arrival time.

Fig. 17 shows the distribution of maximum flood depth evaluated according to Eqs. (20) and (21) for each GCM and SSP. Consistent with the results presented in Fig. 15, the probability distribution of the maximum flood depth for each GCM tends to exhibit higher values in response to less severe climate change scenarios. The average PDF indicated in Figs. 17(a) to (e) was obtained by combining multiple individual PDFs associated with each GCM, assuming equal weights for each distribution. The expected value of the average PDF among GCMs for SSP 2.6, 4.5, 6.0, and 8.5 were evaluated to be 3.20, 3.09, 2.94, and 2.70 m, respectively. Similarly, the variability of the average PDF is lower for a higher SSP. This trend is attributable to similar precipitation patterns among SSPs and lower temperature that decelerates the evapotranspiration rate.

Traditionally, the assessment of maximum flood depth distribution relies on historical local weather records. To effectively evaluate the impact of climate change, these historical records from 1941 to 2020 (covering an 81-year period) can serve as a reference point for comparison, hereafter referred to as the probability of maximum flood depth without climate change effects. This alignment in time allows for a meaningful comparison between GCM simulations spanning from 2021 to 2100 and historical weather records. However, due to the limited availability of data in Fukuyama city, historical simulation data from GCMs are employed as a substitute for historical record data for illustrative purposes. Fig. 17(d) shows the PDF of the maximum flood depth using the historical simulation data of each GCM. It is evident that the average PDF of the maximum flood depth from the historical simulation is notably lower than the future projections, with an expected value of 2.45 m. This distribution is used to estimate the partial factor γRa and γSa to ensure the safety state of the embankment against sliding. In addition, the partial factors to consider the climate change effects (γRb and γSb) were evaluated using the average PDF of the maximum flood depth considering all GCM and SSP (hereafter referred to as the PDF of the maximum flood depth with climate change effects), as shown in Fig. 17(e). Fig. 18 shows the distributions associated with the maximum flood depth Hexc,max for both without and with climate change effects.

## 4.4. Time-dependent failure probability assessment considering the climate change

Details of fragility consideration like structure type, event, class of failure, and the fragility curve for this study, this is an empirical fragility analysis based on the study done in [3.1.7]. here you can briefly explain it like a case study.

Figure: Time-dependent failure probability and the following metrics either in tabular form or other form of illustration which are standard to this research domain

Minimum metrics you should derive

For your paper, I would recommend these as the core set:

- μ(t), σ(t)from BAMLSS

- Time-varying exceedance probability P(Ht>hc)

- Time-varying return level zT(t)

- Annual failure probability Pf(t)

- Life-cycle failure probability PLC

- Expected number of exceedances/failures

- Design-life level

- Covariate contribution to Pf(t)

10. Covariate sensitivity / contribution metrics

This is very important for novelty.

You should quantify:

∂μ∂Xi,∂σ∂Xi

and then:

∂Pf∂Xi

This tells:

Which climate covariate most increases failure probability?

For example:

- ENSO mainly changes μ

- IPO mainly changes σ

- sea-level rise shifts μ

- combined ENSO + SLR produces highest Pf(t)

This is where your framework becomes more than just a curve.

So derive a decomposition like:

| Source | Effect on hazard | Effect on failure probability |
| --- | --- | --- |
| ENSO | changes μ | increases/decreases annual Pf |
| IPO | changes σ | changes tail spread |
| SLR | shifts μupward | raises baseline Pf |
| ENSO + SLR | combined high state | produces reliability-critical years |

This will support the claim that your research gives process-informed reliability understanding, not only nonstationary hazard estimation.

A few line of remarks on overall so what new information we understood from this research and how now ouw thinking should be shifting in understanding the climate state and nonstationary hazard while determining the time-dependent reliability of structures.

Discussion fromt the perpecitve of fragility conditioned on hazard state Answer: which climate covariates causes failure probability growth? Is the increase due to location change? Is it due to scale change? Which climate state produces highest failure probability? How much reliability is lost if climate covariates are ignored?

The analyzed embankment is designed against overtopping and slope instability. This study focuses on developing a design methodology to achieve the target reliability for each limit state without considering the system reliability. Since the consequences of both limit states are relatively similar (i.e., failure of an embankment that causes flooding), the target reliability index βt for both limit state functions is assumed to be the same. For the overtopping failure, the target reliability index βt is used to determine the embankment design height he.

The embankment height design he is then utilized further when evaluating the reliability of embankment against slope instability. It is noteworthy that increasing the target reliability index for overtopping failure can reduce the reliability index associated with slope failure. This is attributed to the requisite design of a higher embankment, which subsequently results in an augmented soil mass, thereby exacerbating the susceptibility to slope instability.

After obtaining the embankment design height he, the reliability index against slope instability βH and the associated MPP X* are evaluated by solving the optimization problem in Eqs. (27) to (30). In this study, the set of random variables involved in the limit state function for slope instability is defined as X = {c’, ϕ’, ρdry, Hexc,max}, with ρw = 10 kN/m3 and ρsat = 1.15ρdry. In addition, the center coordinates of the critical failure surfaces and the corresponding radius are shown in Fig. 19 and were evaluated under an effective cohesion of 5 MPa, a friction angle of 32°, and a slope ns of 2. The random variables associated with the soil parameters and the calculated maximum flood depth are summarized in Table 1. Finally, the design points XD* and the partial factors γXa and γXb were evaluated according to Fig. 6 and Eqs. (1) to (6).

Table 2 shows the reliability assessments and design for the random variables considering several target reliability indexes βt. The increase in the target reliability index βt necessitates a higher embankment design for the overtopping limit state, consequently increasing the volume of sliding material and decreasing the reliability against slope instability. Furthermore, in response to climate change, the embankment height he should be raised since the distribution associated with the maximum flood depth Hexc,max becomes more extreme. Notably, due to the negative correlation between the effective cohesion and the friction angle, the MPP for cohesion is greater than its mean value despite effective cohesion being strongly related to the resistance R in Eq. (24). Focusing on the partial factors to consider climate change effects γXb, the soil parameters exhibits a lower value since these parameters are not directly influenced by climate change. In contrast, the maximum flood depth Hexc,max stands out as the primary parameter significantly affected by climate change. The additional partial factor γXb for the maximum flood depth ranges between 1.31 and 1.40, underscoring its vulnerability to climate-induced changes.

Table 3 shows the partial factors in resistance R and load actions S in the format of Eq. (4). The partial factor γRa applied to the mean value of resistance R to ensure safety is below 1.0, a common notion for LRFD methodology. However, the partial factor γSa for load actions shows a value close to 1.0. This is primarily because the load action S is solely affected by the dry weight ρdry, where this random variable has a considerably lower COV value of 0.1. In addition, the dry weight ρdry appears in both resistance R and load actions S. Nevertheless, the additional partial factors γRb and γSb to accommodate the influence of climate change for both variables are substantially high.

The partial factors in Eq. (4) correspond to the design point considering climate change effects XD,cc*. In addition, the limit state function at the design point g(XD*) and g(XD,cc*) describes the minimum safety margin that should be satisfied for the design, such that g(X) > g(XD,cc*) > 0 [113]. Therefore, the partial factors in Table 3 can be validated based on the design point XD,cc* by performing MCS and counting the frequency of samples that fall within a region bounded by g(XD,cc*) < 0, as suggested by Takenobu et al. [114]. Table 4 shows the validation of the partial factor design in Table 3 based on MCS for several target reliability indices. The value of g(XD,cc*) associated with the safety margin decreases as the target reliability βt increases. In addition, the reliability index estimated from MCS (βt,MCS) indicates a similar result to the target reliability index βt. This result suggests that the proposed method offers a more practical way to evaluate the reliability and partial factors. Furthermore, to visualize the relative location of the characteristic value Xk with respect to the MPP point X* and the design point XD,cc*, Fig. 20 shows the scatter plot of the generated MCS sample associated with the resistance R and load actions S based on Tables 2 and 3. The joint distribution associated with R and S becomes more scattered as the target reliability index increases.

Considering the climate change effects on the design of embankments is of significant importance. The proposed framework could serve as a reference for future code design to consider the impact of non-stationary climate change effects and should be adopted to promote a resilience community against riverine flooding disasters.

# 5. Conclusions

Instruction for writing the conclusion: before writing this make sure to reread again critical the introduction and the research gap, research challenges and significance, relevance and novelty of the study that we claimed in the introduction, then read methodology and result template discussion and probable findings. Based on all this section prepare me five point conclusion remarks for this study which clearly shows the novel understanding that this study provides and the interesting things that this study found. The “blue” highlighted text in this section can be used as reference for writing style. And finally a brief one paragraph explanation on how this study can be extended in future.

Use the following sentence structure and format as writing style

The conclusions resulting from this study are:

- A novel framework for climate change risk adaptation for embankment against non-stationary flooding based on the LRFD approach is established. Future precipitation and temperature projections from various GCMs are bias-corrected using the QDM method to create more accurate climate information. In addition, an integrated hydrological and hydraulic modeling of the analyzed river is carried out to estimate time-variant river discharge and water surface elevation.

- The statistics associated with the flood depth are evaluated by subtracting the water level results from the hydraulic assessment from the river height. Based on these statistics, a combination of the Pareto distribution and extreme value theory is developed to estimate the probability distribution of non-stationary maximum flood depth within a predefined time interval.

- Hydrological and hydraulic analyses reveal that more severe SSP scenarios result in lower streamflow and flood depths. This is driven by a relatively constant trend in annual precipitation across SSP scenarios and higher temperatures in elevated SSPs, leading to increased evapotranspiration. Nevertheless, compared to the historical simulations of GCMs, the PDF of maximum flood depth in the future is expected to be more extreme.

- The reliability assessment and partial factor estimation for river embankment against overtopping and slope instability under non-stationary climate change effects are developed using the 1-σ ellipsoid. The reliability is evaluated by finding the minimum ellipsoid that touches the failure surface based on the SQP algorithm. Due to the effects of climate change, the MPP and design points of each random variable are shifting. Therefore, additional partial factors are introduced to transform the original design points into new design points that take into account the climate change effects.

- The partial factors for climate change impact on maximum flood depth are notably high, while soil parameters suggest a lower factor due to minimal climate change effects. Nonetheless, the partial factors for resistance and load actions are substantially high, emphasizing the importance of considering climate change effects in the embankment design process.

The proposed framework can be extended to address various design problems related to non-stationary climate change effects, such as the degradation of infrastructure components [115–117], roof loads from snow and rain [118], and long-term bridge scouring [119]. The proposed framework should be applied to promote better risk mitigation strategies for communities prone to river flooding. This study focused on evaluating embankment reliability under overtopping and slope instability. Future research should incorporate additional failure modes for more comprehensive reliability assessments and design, such as back erosion piping. Furthermore, future studies should calibrate parameters in hydrological assessments to improve SWAT’s predictive performance.

Acknowledgments

References

Provide intext citation as well as referenes based on the journal guidelines of journal “Structural Safety” or publisher “Elsevier” Fill up the reference that are cited in the manuscript here.