1. What the paper actually developed

The paper studied Hurricane Katrina bridge damage and related observed damage states to coastal hazard intensity measures. The authors considered surge elevation, relative surge, water speed, wind speed, number of spans, and bridge age. Their main finding was that storm surge elevation and number of spans were the strongest predictors of bridge damage, while water speed and 10-minute wind speed showed weaker correlation with increasing damage level.

They then developed:

- Damage-state definitions: slight, moderate, extensive, complete.

- Empirical damage matrices: probability of being in each damage state given a surge bin.

- Exceedance probability matrices: probability of being in or exceeding a damage state given a surge bin.

- Lognormal empirical fragility curves: probability of exceeding moderate, extensive, or complete damage as a function of surge elevation.

The fragility function has the usual form:

P(DS≥dsi∣IM=x)=Φln⁡(x)-ln⁡(θi)βi

where:

P(DS≥dsi∣IM=x)

is the probability of exceeding damage state dsi, IMis storm surge elevation, θiis the median surge causing exceedance of damage state i, βiis logarithmic dispersion, and Φis the standard normal cumulative distribution function.

The paper reports, for example, that the median surge for the moderate damage fragility curve is approximately 7.7 m, with high dispersion.

2. Critical review: what is strong and what is weak

The paper is valuable because it is based on real observed bridge damage, not only simulations. That is its main strength. Empirical fragility is important because analytical models may miss construction details, connection weaknesses, debris effects, and maintenance condition.

However, the paper has serious limitations for direct use in your study.

First, the fragility curves are based mainly on one hurricane event. This means the results are biased by Katrina’s track, surge field, wave characteristics, exposure direction, local bathymetry, and bridge inventory. The authors acknowledge that limited data and storm-event bias introduce uncertainty.

Second, the sample size is small. Although 262 bridges were considered in the reduced exposed population, only 36 damaged bridges in Mississippi and Louisiana were included in the fragility matrix. This is statistically thin, especially for complete damage.

Third, the intensity measure is simplified. The final fragility curves mainly use peak surge elevation, but coastal bridge failure is physically driven by surge, wave height, wave period, velocity, deck clearance, trapped air, buoyancy, drag, uplift, bearing resistance, connection type, and debris impact. The paper notes that relative surge was not feasible for the full inventory because deck elevation data were not reliably available.

Fourth, bridge-class control is incomplete. The authors recognized that bearing type, shear keys, vents, deck elevation, and connection details would improve future models, but these data were not available for all bridges. This is critical. A bridge with shear keys or positive deck-to-substructure connections may behave very differently from a simply supported bridge resting on bearings.

Fifth, the exceedance probabilities are not perfectly monotonic across surge bins. For example, the empirical exceedance probability for extensive damage is 0.172 in the 4.27–5.49 m bin, 0.156 in the 5.49–6.71 m bin, and 0.071 in the 6.71–7.92 m bin. Physically, fragility should generally increase with intensity. This non-monotonicity is a symptom of small sample size, binning effects, and unmodeled bridge differences.

So, use the paper as a methodological template, not as a plug-and-play fragility model unless your bridges are very similar to the Katrina bridge class.

3. Sections you should study carefully before adapting it

You should read the paper in this order.

A. Section 2: bridge damage mechanisms

This section explains the observed failure modes: span shifting, span unseating, bearing damage, guardrail damage, scour, electrical/mechanical damage, and debris impact. It also explains why low-lying simply supported concrete spans were vulnerable to hydrodynamic and hydrostatic forces.

You need this section because your definition of “failure” must be consistent with the physical mechanism. For life-cycle failure probability, “failure” may mean:

DS≥Moderate

or

DS≥Extensive

or

DS=Complete

depending on whether you define failure as service interruption, major repair, or collapse/unseating.

B. Section 3: Data

This is the most important section for replication. It tells you what data were used: reconnaissance damage data, NBI bridge data, FEMA surge estimates, GIS interpolation, and ADCIRC hindcast data for water speed and wind speed.

You must understand this section before adapting the method because your fragility function will be only as reliable as your bridge inventory and hazard data.

C. Table 1: damage-state definitions

This table is essential. The paper defines slight, moderate, extensive, and complete damage. Complete damage includes span unseating, loss of bearing support, imminent deck collapse, or tilting of substructure due to foundation failure.

For your study, do not blindly copy these definitions. Adapt them to your bridge system and research objective. For example:

| Research objective | Recommended failure definition |
| --- | --- |
| Service interruption | DS≥Moderate |
| Major repair / high economic loss | DS≥Extensive |
| Structural collapse / span loss | DS=Completeor DS≥Complete |

D. Section 4.1: hazard-damage analysis

This section explains why the authors selected surge elevation as the main intensity measure. They compared damage rank with surge, relative surge, water speed, and wind speed, and found stronger correlation with peak surge.

For your work, you should repeat this screening. Do not assume surge elevation is always the best intensity measure. For coastal bridges, a better intensity measure may be:

RS=ηsurge-zdeck

where RSis relative surge, ηsurgeis water surface elevation, and zdeckis low chord or deck elevation.

Physically, relative surge is often better than absolute surge because a 4 m surge is dangerous for a bridge with a 2 m deck elevation but less dangerous for a bridge with a 7 m deck elevation.

E. Section 4.2: empirical fragility estimates

This is the core section for your method. It explains how the authors used damaged and undamaged bridges to estimate the probability of exceeding damage states given surge elevation, then fitted lognormal fragility curves.

F. Section 5: conclusions and opportunities

This section is important because it states the paper’s own limitations: need for more bridge subclassification, better bridge parameters, simulated hazard variation, wave height, and joint intensity measures.

4. Step-by-step method for your bridge fragility and life-cycle failure probability

Step 1: Define the bridge class

Do not mix all bridges together. Fragility is class-specific.

Create classes such as:

C1=low-elevation, simply supported, concrete girder, water crossing
C2=continuous concrete bridge with shear keys
C3=steel girder bridge over tidal channel

The Katrina paper focused mainly on low-lying water crossings with concrete superstructures and limited superstructure-substructure connectivity. Therefore, if your bridges are not similar, the Katrina fragility should not be used directly.

Recommended classification variables:

material, span type, number of spans, deck elevation, bearing type, shear keys, connection type, age, scour condition

The paper found number of spans statistically important, so you should consider span-number bins, for example:

Nspan<10,10≤Nspan≤50,Nspan>50

Step 2: Define damage states

Use the paper’s Table 1 as a starting point, but adapt it. A practical version for coastal bridge failure is:

| Damage state | Meaning |
| --- | --- |
| DS0 | No damage |
| DS1 | Slight: minor cracking, minor spalling, small nonstructural damage |
| DS2 | Moderate: bearing damage, guardrail damage, approach settlement, moderate scour, service disruption possible |
| DS3 | Extensive: unsafe components, significant residual displacement, submerged mechanical/electrical equipment, major scour |
| DS4 | Complete: span unseating, loss of bearing support, collapse risk, foundation failure |

For life-cycle failure probability, select a threshold:

F={DS≥DSf}

For example, if failure means major functional loss:

DSf=DS3

Then your fragility is:

Pf(x)=P(DS≥DS3∣IM=x)

Step 3: Select the coastal intensity measure

The paper used peak storm surge elevation because it was available and statistically correlated with damage. But for your study, I recommend testing at least three options:

IM1=ηsurge
IM2=ηsurge-zdeck
IM3=(ηsurge-zdeck,Hs)

where Hsis significant wave height.

If you only use surge elevation, your model may confuse exposure and vulnerability. For example, a bridge with high deck clearance and a bridge with low deck clearance may receive the same surge value, but their failure probabilities are not the same.

Best practical recommendation:

Use relative surge as the primary IM if you can obtain deck elevation:

RS=ηsurge-zlow chord

If RS<0, water does not reach the deck. If RS>0, buoyancy, uplift, wave impact, and debris effects become much more serious.

Step 4: Build your empirical dataset

For every bridge j, create one row:

IDjDSjIMjNspan,jzdeck,jmaterialjconnectionjagejlocationj

You need both damaged and undamaged bridges. This is essential. The Katrina paper compared damaged bridge locations with undamaged bridges in the exposed region to estimate conditional damage probabilities.

Your dataset should look like this:

| Bridge | Surge m | Relative surge m | Damage state | Spans | Type |
| --- | --- | --- | --- | --- | --- |
| B1 | 3.5 | 0.8 | DS0 | 12 | RC simply supported |
| B2 | 4.2 | 1.5 | DS2 | 34 | RC simply supported |
| B3 | 5.6 | 2.1 | DS3 | 80 | RC simply supported |
| B4 | 6.4 | 3.0 | DS4 | 120 | RC simply supported |

Step 5: Create damage probability matrices

Bin the intensity measure. The paper used surge bins:

0.61-1.83,1.83-3.05,3.05-4.27,4.27-5.49,5.49-6.71,6.71-7.92 m

For each bin:

P(DS=k∣IM∈bin)=n(DS=k,IM∈bin)n(IM∈bin)

Using the paper’s Table 3, in the 3.05–4.27 m surge bin:

ntotal=43
nmoderate=3,nextensive=4,ncomplete=1

So:

P(DS=Moderate∣3.05<IM<4.27)=343=0.070
P(DS=Extensive∣3.05<IM<4.27)=443=0.093
P(DS=Complete∣3.05<IM<4.27)=143=0.023

These are exactly the type of probabilities shown in the paper’s damage probability matrix.

Step 6: Convert to exceedance probabilities

Fragility usually uses exceedance probability:

P(DS≥dsi∣IM=x)

For example, using the same 3.05–4.27 m bin:

P(DS≥Moderate)=nModerate+nExtensive+nCompletentotal
P(DS≥Moderate)=3+4+143=0.186

Similarly:

P(DS≥Extensive)=4+143=0.116
P(DS≥Complete)=143=0.023

These match the paper’s exceedance matrix.

This step is the bridge between empirical damage observations and fragility curves.

Step 7: Fit a lognormal fragility curve

For each damage threshold DS≥dsi, fit:

P(DS≥dsi∣IM=x)=Φln⁡(x)-ln⁡(θi)βi

Use your empirical points:

xbpb

where xbis the midpoint of each intensity bin and pbis the exceedance probability.

For the 3.05–4.27 m bin:

xb=3.05+4.272=3.66 m

For DS≥Moderate:

pb=0.186

Transform:

Φ-1(pb)=ln⁡(xb)-ln⁡(θ)β

Then regress:

ln⁡(xb)=ln⁡(θ)+βΦ-1(pb)

The intercept gives:

ln⁡(θ)

and the slope gives:

β

This is the same basic procedure used in the paper, where the authors fitted lognormal fragility curves using simple linear regression.

Step 8: Choose your failure limit state

For life-cycle failure probability, define failure carefully.

Common choices:

Pf(x)=P(DS≥Moderate∣x)

for service disruption;

Pf(x)=P(DS≥Extensive∣x)

for major repair or severe functionality loss;

Pf(x)=P(DS≥Complete∣x)

for collapse or span unseating.

For bridge life-cycle safety, I recommend using at least two thresholds:

DS≥Extensive

as “functional failure,” and

DS≥Complete

as “structural failure.”

This gives a more defensible study because agencies may care about both closure probability and collapse probability.

Numerical example

Suppose your fitted fragility for DS≥Extensivegives:

| Surge bin midpoint xi | Fragility Pf(xi) | Annual probability of surge in bin Δλi |
| --- | --- | --- |
| 2 m | 0.05 | 0.020 |
| 4 m | 0.15 | 0.010 |
| 6 m | 0.35 | 0.004 |
| 8 m | 0.60 | 0.001 |

Then:

Pf,annual=0.05(0.020)+0.15(0.010)+0.35(0.004)+0.60(0.001)
Pf,annual=0.001+0.0015+0.0014+0.0006=0.0045

So:

Pf,annual=0.45%

For a service life of T=75years, assuming independent annual events and stationary hazard:

Pf,life=1-(1-Pf,annual)T
Pf,life=1-(1-0.0045)75
Pf,life=1-0.995575
Pf,life≈0.287

So the 75-year failure probability is approximately:

28.7%

If climate change, do not use a constant annual probability. Use:

Pf,life=1-t=1T1-Pf,t

where Pf,tchanges with time.

For example, if sea-level rise increases relative surge over time:

RSt=ηsurge,t+SLRt-zdeck,t

then your annual fragility becomes time-dependent:

Pf,t(x)=Φln⁡(x)-ln⁡(θt)β

6. How you should adapt the Katrina paper to your study

Recommended workflow

- Define bridge class
Start with one homogeneous class. For example: low-lying concrete simply supported coastal bridges.

- Define damage/failure states
Use Table 1 from the paper, but adapt it to your target consequence: closure, major repair, collapse, or span unseating.

- Collect bridge inventory data
Minimum data: location, deck elevation, low-chord elevation, number of spans, span length, material, bearing type, connection type, age, foundation type, scour rating, and maintenance condition.

- Collect coastal hazard data
Use storm surge, wave height, wave period, current velocity, wind speed, and debris exposure if available.

- Compute bridge-specific intensity measures
Prefer relative surge:

RS=ηsurge-zdeck

- Create empirical damage matrix
Count damaged and undamaged bridges in each intensity bin.

- Create exceedance matrix
Convert damage counts into:

P(DS≥dsi∣IM)

- Fit fragility curves
Fit lognormal, probit, or logistic fragility models.

- Check physical consistency
Fragility should generally increase with intensity. If not, investigate bin size, sample size, or missing explanatory variables.

7. My recommendation for your empirical fragility model

For your intended study, I would not simply copy Figure 10 from the paper. I would develop your own model in this hierarchy:

Level 1: Simple empirical fragility

P(DS≥dsi∣RS)=Φln⁡(RS)-ln⁡(θi)βi

Use this if your data are limited.

Level 3: Time-dependent life-cycle fragility

Use:

P(DS≥dsi∣IM,t)

where covariates affects hazard intensity level and thereby influence the fragility.

This is the most suitable form for life-cycle failure probability.